##### You said:

Based upon this please creata guideline summary per class type for Category A in Riyadh

###### ChatGPT said:

Below is a Category A – Riyadh (Low-Rise School) Guideline Summary per Class Type, synthesized from:

- Handbook of Typical School Design – 2 & 3 Classroom Blocks

199270_1

- Design Principles for Schools – Learning Policy Institute

SoLD_Design_Principles_REPORT

- The Design of School Buildings – Informal Learning Spaces

The design of school buildings

Adapted specifically for Riyadh climate (hot desert), MOE Category A compliance, low-rise campus model, and aligned with safe, child-friendly, whole-child principles.

# 📘 CATEGORY A – RIYADH

## Guideline Summary per Class Type

# 1️⃣ EARLY YEARS (Nursery / KG1 / KG2)

### 🔹 Recommended Area

- 60–70 m² minimum

- Ceiling height: ≥ 3.2 m

- Direct outdoor access mandatory

### 🔹 Layout & Zoning

- Activity-based learning zones:

  - Reading corner

  - Creative/art zone

  - Soft play

  - Circle time

- Washbasin inside classroom (low height)

- Direct access to shaded outdoor play area

### 🔹 Environmental Design (Riyadh Adaptation)

- Orientation: Long façade North–South to reduce heat gain

199270_1

- Deep shaded verandah (≥ 2.5 m)

199270_1

- Double glazing + thermal insulation

- Mechanical ventilation + filtered fresh air

- Anti-slip flooring

### 🔹 Safety & Child-Friendly Features

- Rounded furniture edges

199270_1

- Emergency exit door opening outward

199270_1

- Ramp access instead of stairs

199270_1

- Visual supervision lines

### 🔹 Whole Child Design Alignment

- Emotional safety + belonging focus

SoLD_Design_Principles_REPORT

- Play-based, inquiry-driven environments

# 2️⃣ PRIMARY CLASSROOMS (Grade 1–4)

### 🔹 Recommended Area

- 56–65 m²

- Based on 7.0 m × 8.0 m prototype

199270_1

- Ceiling height: 3.5 m preferred

199270_1

### 🔹 Capacity

- 20–25 students

### 🔹 Layout

- Flexible furniture (clusterable desks)

- Display walls for student work

- Reading/library shelf inside room

- Terrace access (2.5 m width recommended)

199270_1

### 🔹 Environmental Requirements

- Cross ventilation schematic principle

199270_1

- External shading devices (louvers/pergola)

199270_1

- LED lighting plan per classroom electrical layout

199270_1

### 🔹 Learning Design

- Support differentiated learning modes

SoLD_Design_Principles_REPORT

- Visible supervision in corridors

- Informal spill-out space for peer collaboration

The design of school buildings

# 3️⃣ UPPER PRIMARY / JUNIOR (Grade 5–6)

### 🔹 Recommended Area

- 60–70 m²

### 🔹 Capacity

- 25–28 students

### 🔹 Layout Features

- Collaborative tables

- Writable walls

- Technology integration points

- Adjacent informal breakout space

The design of school buildings

### 🔹 Design Emphasis

- Encourage self-directed learning in corridors / assembly spaces

The design of school buildings

- Moderate acoustic treatment

### 🔹 Developmental Alignment

- Skills, habits, and mindset development

SoLD_Design_Principles_REPORT

- Structured yet flexible learning zones

# 4️⃣ MIDDLE SCHOOL (Grade 7–9)

### 🔹 Recommended Area

- 65–75 m²

### 🔹 Capacity

- 28–30 students

### 🔹 Layout Requirements

- Modular furniture

- Digital display integration

- STEM capability (power/data points)

- Shared lab access

### 🔹 Informal Learning Integration

- Wide corridors / learning commons

The design of school buildings

- Assembly spaces for peer learning

The design of school buildings

### 🔹 Environmental

- Higher acoustic insulation

- Enhanced daylight control

### 🔹 Whole Child Alignment

- Rich learning experiences

SoLD_Design_Principles_REPORT

- Culturally responsive learning spaces

# 5️⃣ SECONDARY (Grade 10–12)

### 🔹 Recommended Area

- 70–80 m²

### 🔹 Capacity

- 30 students max

### 🔹 Layout

- Seminar-style configuration possible

- Lab-ready electrical grid

- Storage + display for projects

### 🔹 Specialized Rooms

- Science lab (85–100 m²)

- ICT Lab (80 m²)

- Multipurpose hall convertible use

199270_1

### 🔹 Learning Model

- Multidisciplinary teaching spaces

SoLD_Design_Principles_REPORT

- Strong framing + defined learning zones

The design of school buildings

# 🏫 SHARED SPACES – CATEGORY A REQUIREMENTS

### 🔹 Toilets

- Separate boys/girls blocks

199270_1

- Hygiene & handwashing stations

199270_1

### 🔹 Multipurpose Hall

- Convertible classroom division

199270_1

- Assembly + sports use

### 🔹 Outdoor Areas

- North–South oriented playfield

199270_1

- Shaded courtyards

- No undefined hidden spaces

199270_1

# 🌡️ RIYADH-SPECIFIC ADAPTATIONS

| Parameter | Requirement |
| --- | --- |
| Climate | Extreme heat mitigation |
| Insulation | High-performance envelope |
| Roof | Lightweight, reflective 199270_1 |
| Setback | Noise + privacy buffer 199270_1 |
| Solar | Shading mandatory |
| Drainage | Closed drainage system 199270_1 |

# 🛡️ STRUCTURAL & SAFETY – CATEGORY A

- Simple rectangular plan for stability

199270_1

- Proper structural detailing

199270_1

- Outward-opening doors

199270_1

- Emergency escape ramp per classroom

199270_1

# 📊 SUMMARY TABLE

| Class Type | Area (m²) | Key Design Feature | Riyadh Adaptation |
| --- | --- | --- | --- |
| Early Years | 60–70 | Play zones + outdoor access | Full shading |
| Primary | 56–65 | Flexible clusters | Cross ventilation |
| Upper Primary | 60–70 | Writable walls | Breakout space |
| Middle | 65–75 | Modular + STEM ready | Acoustic control |
| Secondary | 70–80 | Seminar / lab ready | Solar control |

# 🎯 Final Recommendation for PISES (Category A – Riyadh)

For a modern Category A campus in Riyadh:

- Low-rise blocks with shaded internal courtyards

- Classroom size scalable 60–75 m² standard

- Informal learning corridors (not dead corridors)

The design of school buildings

- Whole-child design embedded in spatial planning

SoLD_Design_Principles_REPORT

- Climate-first façade design

If you would like, I can now:

- Convert this into a formal Basis of Design section (Folder 04 format)

- Or create a MOE-ready compliance matrix table

- Or create a Gamma.app slide deck for SMC presentation

Let me know which direction you want to take.

Sources

##### You said:

use the TBC document called english manual and extract data from it

###### ChatGPT said:

Based on the TBC – Manual of the Minimum Design Criteria for Private Schools (English Manual)

EnglishManual

and focusing on Category A (Type A cities – Riyadh, Dammam, Alkhobar)

EnglishManual

, below is a structured Guideline Summary per Class Type applicable to Riyadh (Category A).

# 📘 CATEGORY A (RIYADH) – DESIGN GUIDELINE SUMMARY PER CLASS TYPE

(Extracted from TBC English Manual)

## 1️⃣ Nursery

(From Nursery criteria section – general principles apply; area values based on geographic category)

### 👶 Area Standards

- Infant room: 1.5 m² per 10 infants (special room)

EnglishManual

- WC: 1 WC per 15 students, 1.8 m² each

EnglishManual

- Outdoor emergency court: ½ of activity room area

EnglishManual

### 🏫 Design Principles

- Ground level priority

- Emergency outdoor access

- Age-appropriate sanitary fixtures

- Slip-resistant stairs

## 2️⃣ Kindergarten (Category A – Riyadh)

### 👦 Maximum Students

- Max 20 students per classroom

EnglishManual

### 📐 Area per Child (Category A)

- 2.5 m² per child

EnglishManual

### 🚪 Outdoor Court

- Each classroom must have outdoor court

- Area = ½ classroom area

EnglishManual

### 🎠 Playing Court

- Total area = Double classroom area

EnglishManual

### 🚻 Toilets

- One toilet inside each classroom

- 1.8 m² minimum per toilet

EnglishManual

### 📚 Mandatory Facilities

- Classrooms

- Toilets

- Outdoor court per classroom

- Multimedia library

EnglishManual

## 3️⃣ Elementary (Category A – Riyadh)

### 👦 Maximum Students

- Max 25 students per classroom

EnglishManual

### 📐 Classroom Area (Category A)

- 1.3 m² per student

EnglishManual

  - 1.8 m minimum distance to board

EnglishManual

- Max dimensional ratio: 2.3

EnglishManual

### 🔬 General Science Lab

- 1.9 m² per student

EnglishManual

- Max 25 students

- 1 lab per 10 classrooms

### 💻 Computer & Language Lab

- 1.9 m² per student

EnglishManual

- Max 25 students

### 🎨 Vocational Rooms (Upper Elementary – Girls)

- Arts / Sewing / Home Economics

- Max 25 students

EnglishManual

### 🏀 Outdoor Court

- Minimum 300 m²

EnglishManual

## 4️⃣ Intermediate (Category A – Riyadh)

### 👦 Maximum Students

- Max 25 students per classroom

EnglishManual

### 📐 Classroom Area

- 1.3 m² per student

EnglishManual

- 1.8 m board clearance

- Max ratio 2.3

### 🔬 General Science Lab

- 2.0 m² per student

EnglishManual

- 1 lab per 10 classrooms

EnglishManual

### 💻 Computer & Language Lab

- 2.0 m² per student

EnglishManual

### 🎨 Arts Atelier

- 1.35 m² per student

EnglishManual

### 🏀 Outdoor Pitch

- Minimum 400 m²

EnglishManual

## 5️⃣ Secondary (Category A – Riyadh)

### 👦 Maximum Students

- Max 25 students per classroom

EnglishManual

### 📐 Classroom Area

- 1.3 m² per student

EnglishManual

- 1.8 m board clearance

- Max ratio 2.3

### 🔬 Science Labs (Physics, Chemistry, Biology)

- 2.25 m² per student

EnglishManual

- Chemistry lab requires preparation room

- Emergency exit required

EnglishManual

### 💻 Computer & Language Lab

- 2.25 m² per student

EnglishManual

### 🎨 Arts & Sewing (Vocational)

- 1.5 m² per student

EnglishManual

### 🏀 Outdoor Pitch

- Minimum 400 m²

EnglishManual

# 🔎 Key Universal Rules (All Levels – Category A)

✔ Maximum 25 students (except KG = 20)

EnglishManual

✔ Minimum 1.8 m from first desk to board

EnglishManual

✔ Max dimension ratio = 2.3

EnglishManual

✔ Natural light & ventilation mandatory

EnglishManual

✔ Accessibility compliance required

EnglishManual

# 📊 Riyadh (Category A) Quick Reference Table

| Level | Classroom m² / Student | Lab m² / Student | Max Students | Outdoor |
| --- | --- | --- | --- | --- |
| Nursery | Activity-based | — | Age-based | ½ room |
| Kindergarten | 2.5 | — | 20 | ½ + Double play |
| Elementary | 1.3 | 1.9 | 25 | 300 m² |
| Intermediate | 1.3 | 2.0 | 25 | 400 m² |
| Secondary | 1.3 | 2.25 | 25 | 400 m² |

If you would like, I can now:

- Convert this into a Basis of Design – Category A (Riyadh) compliance matrix

- Or format it as a Gamma.app slide deck

- Or build it as a Teams-ready folder document structure for PISES new building

Sources

##### You said:

Class Section Name Strength Name Boys Girls Total 2024-25 Nursery 122 (Boys:66 Girls:56) A (Morning) 8 12 20 B (Morning) 5 15 20 C (Morning) 16 6 22 N (Afternoon) 10 9 19 O (Afternoon) 14 6 20 P (Afternoon) 13 8 21 Reception 296 (Boys:146 Girls:150) A (Morning) 9 14 23 B (Morning) 13 10 23 C (Morning) 6 16 22 D (Morning) 9 11 20 E (Morning) 12 12 24 F (Morning) 13 9 22 G (Morning) 13 9 22 N (Afternoon) 12 8 20 O (Afternoon) 11 8 19 P (Afternoon) 10 9 19 Q (Afternoon) 8 13 21 R (Afternoon) 10 11 21 S (Afternoon) 12 8 20 T (Afternoon) 8 12 20 KG 383 (Boys:203 Girls:180) A (Morning) 10 13 23 B (Morning) 13 7 20 C (Morning) 10 12 22 D (Morning) 9 11 20 E (Morning) 10 12 22 F (Morning) 13 7 20 G (Morning) 10 9 19 H (Morning) 12 10 22 I (Morning) 14 9 23 N (Afternoon) 12 8 20 O (Afternoon) 12 8 20 P (Afternoon) 10 12 22 Q (Afternoon) 11 11 22 R (Afternoon) 12 9 21 S (Afternoon) 9 12 21 T (Afternoon) 9 14 23 U (Afternoon) 15 5 20 V (Afternoon) 12 11 23 Grade-1 394 (Boys:197 Girls:197) A (Morning) 10 13 23 B (Morning) 12 10 22 C (Morning) 14 8 22 D (Morning) 9 11 20 E (Morning) 7 15 22 F (Morning) 12 10 22 G (Morning) 14 7 21 H (Morning) 14 7 21 I (Morning) 10 9 19 N (Afternoon) 9 11 20 O (Afternoon) 9 13 22 P (Afternoon) 12 10 22 Q (Afternoon) 9 10 19 R (Afternoon) 13 6 19 S (Afternoon) 10 10 20 T (Afternoon) 9 12 21 U (Afternoon) 10 11 21 V (Afternoon) 7 13 20 W (Afternoon) 7 11 18 Grade-2 417 (Boys:228 Girls:189) A (Girls) 0 23 23 B (Girls) 0 19 19 C (Girls) 0 23 23 D (Girls) 0 19 19 E (Boys) 20 0 20 F (Boys) 20 0 20 G (Boys) 23 0 23 H (Boys) 18 0 18 I (Boys) 18 0 18 J (Boys) 17 0 17 N (Girls_AN) 0 19 19 O (Girls_AN) 0 20 20 P (Girls_AN) 0 23 23 Q (Girls_AN) 0 22 22 R (Girls_AN) 0 21 21 S (Boys_AN) 22 0 22 T (Boys_AN) 21 0 21 U (Boys_AN) 17 0 17 V (Boys_AN) 18 0 18 W (Boys_AN) 18 0 18 X (Boys_AN) 16 0 16 Grade-3 398 (Boys:206 Girls:192) A (Girls) 0 21 21 B (Girls) 0 17 17 C (Girls) 0 20 20 D (Girls) 0 20 20 E (Girls) 0 14 14 F (Boys) 22 0 22 G (Boys) 21 0 21 H (Boys) 22 0 22 I (Boys) 20 0 20 J (Boys) 22 0 22 N (Girls_AN) 0 21 21 O (Girls_AN) 0 21 21 P (Girls_AN) 0 18 18 Q (Girls_AN) 0 21 21 R (Girls_AN) 0 19 19 S (Boys_AN) 17 0 17 T (Boys_AN) 17 0 17 U (Boys_AN) 17 0 17 V (Boys_AN) 18 0 18 W (Boys_AN) 15 0 15 X (Boys_AN) 15 0 15 Grade-4 430 (Boys:208 Girls:222) A (Girls) 0 20 20 B (Girls) 0 20 20 C (Girls) 0 21 21 D (Girls) 0 18 18 E (Girls) 0 18 18 F (Girls) 0 16 16 G (Boys) 22 0 22 H (Boys) 22 0 22 I (Boys) 23 0 23 J (Boys) 19 0 19 N (Girls_AN) 0 20 20 O (Girls_AN) 0 19 19 P (Girls_AN) 0 18 18 Q (Girls_AN) 0 17 17 R (Girls_AN) 0 19 19 S (Girls_AN) 0 16 16 T (Boys_AN) 18 0 18 U (Boys_AN) 18 0 18 V (Boys_AN) 22 0 22 W (Boys_AN) 21 0 21 X (Boys_AN) 22 0 22 Y (Boys_AN) 21 0 21 Grade-5 422 (Boys:215 Girls:207) A (Boys) 20 0 20 A (Girls) 0 21 21 B (Boys) 22 0 22 B (Girls) 0 21 21 C (Boys) 20 0 20 C (Girls) 0 19 19 D (Boys) 20 0 20 D (Girls) 0 18 18 E (Boys) 20 0 20 E (Girls) 0 21 21 N (Boys_AN) 19 0 19 N (Girls_AN) 0 21 21 O (Boys_AN) 19 0 19 O (Girls_AN) 0 19 19 P (Boys_AN) 19 0 19 P (Girls_AN) 0 24 24 Q (Boys_AN) 19 0 19 Q (Girls_AN) 0 20 20 R (Boys_AN) 19 0 19 R (Girls_AN) 0 23 23 S (Boys_AN) 18 0 18 Grade-6 468 (Boys:227 Girls:241) A (Boys) 18 0 18 A (Girls) 0 20 20 B (Boys) 19 0 19 B (Girls) 0 18 18 C (Boys) 20 0 20 C (Girls) 0 19 19 D (Boys) 20 0 20 D (Girls) 0 22 22 E (Boys) 19 0 19 E (Girls) 0 22 22 F (Girls) 0 21 21 N (Boys_AN) 22 0 22 N (Girls_AN) 0 17 17 O (Boys_AN) 23 0 23 O (Girls_AN) 0 18 18 P (Boys_AN) 22 0 22 P (Girls_AN) 0 21 21 Q (Boys_AN) 22 0 22 Q (Girls_AN) 0 23 23 R (Boys_AN) 20 0 20 R (Girls_AN) 0 22 22 S (Boys_AN) 22 0 22 S (Girls_AN) 0 18 18 Grade-7 429 (Boys:235 Girls:194) A (Boys) 24 0 24 A (Girls) 0 21 21 B (Boys) 19 0 19 B (Girls) 0 21 21 C (Boys) 24 0 24 C (Girls) 0 19 19 D (Boys) 20 0 20 D (Girls) 0 15 15 E (Boys) 20 0 20 E (Girls) 0 22 22 N (Boys_AN) 19 0 19 N (Girls_AN) 0 20 20 O (Boys_AN) 17 0 17 O (Girls_AN) 0 19 19 P (Boys_AN) 19 0 19 P (Girls_AN) 0 17 17 Q (Boys_AN) 19 0 19 Q (Girls_AN) 0 19 19 R (Boys_AN) 18 0 18 R (Girls_AN) 0 21 21 S (Boys_AN) 16 0 16 T (Boys_AN) 20 0 20 Grade-8 381 (Boys:182 Girls:199) A (Boys) 25 0 25 A (Girls) 0 22 22 B (Boys) 23 0 23 B (Girls) 0 21 21 C (Boys) 18 0 18 C (Girls) 0 24 24 D (Boys) 19 0 19 D (Girls) 0 19 19 E (Girls) 0 24 24 N (Boys_AN) 23 0 23 N (Girls_AN) 0 24 24 O (Boys_AN) 22 0 22 O (Girls_AN) 0 21 21 P (Boys_AN) 19 0 19 P (Girls_AN) 0 23 23 Q (Boys_AN) 17 0 17 Q (Girls_AN) 0 21 21 R (Boys_AN) 16 0 16 Grade-9 351 (Boys:183 Girls:168) A (Boys) 24 0 24 A (Girls) 0 19 19 B (Boys) 24 0 24 B (Girls) 0 19 19 C (Boys) 18 0 18 C (Girls) 0 23 23 D (Boys) 18 0 18 D (Girls) 0 17 17 E (Boys) 16 0 16 E (Girls) 0 22 22 N (Boys_AN) 21 0 21 N (Girls_AN) 0 20 20 O (Boys_AN) 17 0 17 O (Girls_AN) 0 17 17 P (Boys_AN) 21 0 21 P (Girls_AN) 0 16 16 Q (Boys_AN) 24 0 24 Q (Girls_AN) 0 15 15 Grade-10 330 (Boys:177 Girls:153) A (Boys) 20 0 20 A (Girls) 0 20 20 B (Boys) 26 0 26 B (Girls) 0 17 17 C (Boys) 19 0 19 C (Girls) 0 18 18 D (Boys) 25 0 25 D (Girls) 0 20 20 E (Boys) 18 0 18 E (Girls) 0 20 20 N (Boys_AN) 23 0 23 N (Girls_AN) 0 17 17 O (Boys_AN) 24 0 24 O (Girls_AN) 0 20 20 P (Boys_AN) 22 0 22 P (Girls_AN) 0 21 21 Grade-11 253 (Boys:116 Girls:137) A (Boys) 21 0 21 A (Girls) 0 21 21 B (Boys) 26 0 26 B (Girls) 0 20 20 C (Boys) 25 0 25 C (Girls) 0 25 25 D (Girls) 0 26 26 N (Boys_AN) 22 0 22 N (Girls_AN) 0 22 22 O (Boys_AN) 22 0 22 O (Girls_AN) 0 23 23 Grade-12 189 (Boys:101 Girls:88) A (Boys) 22 0 22 A (Girls) 0 20 20 B (Boys) 18 0 18 B (Girls) 0 15 15 C (Boys) 13 0 13 C (Girls) 0 17 17 D (Boys) 25 0 25 D (Girls) 0 18 18 N (Boys_AN) 23 0 23 N (Girls_AN) 0 18 18 Total 2690 2573 5263". extrapolating the current number to 7000 how many classroom and other activity room including SEN rooms are needed and what will be size of each and all types of room based upon the TBC guidleins we will have coeduation till grade 1 and then same area till grade 4 but boys and girls will be in seperate classes and then from grade 5 till grade 12 we will have seperate class rooms

###### ChatGPT said:

Thought for 3m 5s

According to a document (TBC English Manual – Manual of the Minimum Design Criteria for Private Schools and Approval Procedures, 1st Edition), below is a Category A (Riyadh) guideline summary per class/space type, and then an extrapolation from your current 5,263 students to 7,000 with the required number of classrooms + key activity rooms.

## 1) Category A space guidelines (by education stage / room type)

### A. Nursery (infants up to 3 years)

Core spaces (required): Activity room, bedroom, sanitary facilities, feeding room, outdoor court (emergency exit).

EnglishManual

- Activity room area per child (Category A):

  - < 1 year: 1.4 m²/child

  - 1 to 3 years: 1.8 m²/child

EnglishManual

- Bedroom area per child (Category A):

  - < 1 year: 0.7 m²/child

  - 1 to 3 years: 0.9 m²/child

EnglishManual

- Feeding room: 1.5 m² per 10 infants

EnglishManual

- Sanitary: 1 WC (1.8 m²) per 15 students

EnglishManual

- Outdoor court (emergency exit): ½ the activity-room area

EnglishManual

### B. Kindergarten (your Reception + KG)

General: spaces at ground level; each classroom at ground level to have an outdoor court as emergency exit.

EnglishManual

- Classroom area per child (Category A): 2.5 m²/child

EnglishManual

- Toilets:

  - 1 kids’ toilet inside each classroom; 1.8 m² per toilet

EnglishManual

  - Public toilet: 1 toilet per 20 persons

EnglishManual

- Outdoor court per classroom (emergency exit): ½ the classroom area

EnglishManual

- Multimedia library: mandatory at every kindergarten (area at investor discretion).

EnglishManual

- Playing court: double the classroom area (for that education level).

EnglishManual

### C. Elementary (Grades 1–6)

Max students per classroom: ≤ 25

EnglishManual

Compulsory facilities include: classrooms, general science lab, computer & languages lab, arts atelier, learning resources center, toilets, multi-purpose room, outdoor pitch/court.

EnglishManual

#### 1) Standard classroom (Category A)

- Min area per student: 1.3 m²/student (Category A) + board clearance rule

EnglishManual

- The manual’s worked example for 25 students gives:
1.3×25 + 1.8×5.90 = 43.12 m²

EnglishManual

✅ Use 43.12 m² as the minimum “standard classroom” baseline (25 students).

#### 2) General science lab (Elementary)

- 1 lab per 10 classrooms

EnglishManual

- Max 25 students per lab

EnglishManual

#### 3) Learning Resources Center (Elementary)

- Category A: 2.5 m²/student + circulation; example: 75.1 m² for 25 students

EnglishManual

#### 4) Toilets (Elementary)

- 1 toilet per 20 students (minimum)

EnglishManual

- Min toilet area: 1.35 m²; disabled toilet 3.15 m² and 1 disabled toilet per cluster

EnglishManual

#### 5) Multi-purpose room (Elementary)

- Max 25 students, teacher area min 8 m²

EnglishManual

- Category A: 1.25 m²/student; example 42.50 m² (25 students)

EnglishManual

#### 6) Outdoor pitch / court

- Outdoor pitch minimum area: 300 m²

EnglishManual

### D. Intermediate (Grades 7–9)

- Max students per classroom: ≤ 25

EnglishManual

- Classroom area/student (Category A): 1.3 m²/student

EnglishManual

- LRC (Intermediate): 2.7 m²/student; example 74.70 m²

EnglishManual

- Multi-purpose (Intermediate): 1.35 m²/student; example 45.90 m²

EnglishManual

- Toilets: same rule 1 per 20 students, min areas etc.

EnglishManual

- Outdoor pitch: included as compulsory (min pitch 300 m² used widely in manual).

EnglishManual

EnglishManual

### E. Secondary (Grades 10–12)

- LRC (Secondary): 3.0 m²/student; example 88.68 m²

EnglishManual

- Multi-purpose (Secondary): 1.5 m²/student; example 51 m²

EnglishManual

- Toilets: same rule 1 per 20 students, min areas etc.

EnglishManual

- Outdoor court minimum area: 400 m²

EnglishManual

## 2) Extrapolation to 7,000 students: how many classrooms?

Your current total is 5,263 students (as provided). Scaling to 7,000 is a factor of ~1.33.

### Classroom planning assumptions used (aligned to the manual)

- Nursery / Reception / KG: planned at 20 students per room (your current sections are around 19–23; and KG is activity-based).

- Grades 1–12: 25 students per classroom max (explicit in manual).

EnglishManual

### Result: required sections (classrooms) at 7,000

- Nursery: 9 rooms

- Reception: 20 rooms

- KG: 26 rooms

- Grade 1: 21 rooms

- Grade 2: 23 rooms

- Grade 3: 22 rooms

- Grade 4: 23 rooms

- Grade 5: 23 rooms

- Grade 6: 25 rooms

- Grade 7: 23 rooms

- Grade 8: 21 rooms

- Grade 9: 19 rooms

- Grade 10: 18 rooms

- Grade 11: 14 rooms

- Grade 12: 11 rooms

✅ Total classrooms = 298

### Co-education / segregation implication (your rule)

- Co-ed: Nursery, Reception, KG, Grade 1 (still 298 total rooms, but shared block)

- Separate boys/girls classrooms: Grade 2–12 (you’ll want two mirrored wings for those grades, even if some shared specialist spaces are centralized with controlled scheduling).

## 3) “Other activity rooms” required (minimum counts driven by TBC ratios where stated)

### General science labs (explicit ratio in manual)

Manual rule: 1 general science lab per 10 classrooms (elementary; same concept repeats by stage).

EnglishManual

Using your 7,000-student classroom counts by stage:

- Elementary classrooms (Grades 1–6): 137 → 14 science labs

- Intermediate classrooms (Grades 7–9): 63 → 7 science labs

- Secondary classrooms (Grades 10–12): 43 → 5 science labs (if the same “per 10 classrooms” planning logic is applied consistently; secondary section also typically needs subject labs—Physics/Chem/Bio—so we can map these 5 into discipline labs in the next step.)

If you want, I can now extract the secondary Physics/Chem/Bio lab pages from the manual and convert them into an exact count + size schedule.

## 4) Minimum room sizes to use in the space program (Category A)

Here are the design-minimums you can paste into the Basis of Design:

### Nursery (typical)

- Activity room: 1.8 m²/child (age 1–3)

EnglishManual

- Bedroom: 0.9 m²/child (age 1–3)

EnglishManual

- Feeding: 1.5 m² per 10 infants

EnglishManual

- WC: 1.8 m² per WC; 1 per 15

EnglishManual

- Outdoor: ½ activity room area

EnglishManual

### Kindergarten (Reception / KG)

- Classroom: 2.5 m²/child

EnglishManual

- Toilet in each classroom: 1.8 m²

EnglishManual

- Outdoor court per classroom: ½ classroom area

EnglishManual

- Playing court: 2× classroom area

EnglishManual

### Grades 1–6 (Elementary) standard classroom (25 students)

- Classroom minimum example: 43.12 m²

EnglishManual

### Grades 7–9 (Intermediate) standard classroom (25 students)

- (Same 1.3 m²/student rule applies)

EnglishManual

### Multi-purpose rooms (per 25 students)

- Elementary example: 42.50 m²

EnglishManual

- Intermediate example: 45.90 m²

EnglishManual

- Secondary example: 51 m²

EnglishManual

### Learning Resources Center (per 25 students)

- Elementary example: 75.1 m²

EnglishManual

- Intermediate example: 74.70 m²

EnglishManual

- Secondary example: 88.68 m²

EnglishManual

### Toilets (Grades 1–12)

- 1 per 20 students minimum, min areas: 1.35 m² standard, 3.15 m² accessible; 1 accessible per cluster

EnglishManual

### Outdoor sports

- Outdoor pitch ≥ 300 m² (elementary)

EnglishManual

- Secondary outdoor court minimum 400 m²

EnglishManual

## 5) Land plot size check (Category A, Riyadh)

For a school complex (multiple levels), the manual gives Category A average land-plot criterion 4.8 m²/student (derived from the level criteria).

EnglishManual

- Minimum land plot for 7,000 students:
7,000 × 4.8 = 33,600 m²

(This is a minimum planning check; actual land will rise with segregation, circulation, parking, admin, halls, canteen, clinics, MEP yards, etc.)

## 6) SEN rooms (important note)

The extracted TBC manual pages above do not state explicit SEN room counts or sizes. So for SEN, the best approach is:

- Keep them as a PISES “Educational Intent / Inclusion” requirement (Folder 02 & Folder 03), and

- Size them using international best practice, then validate with TBC during concept submission.

If you want, I can propose a practical SEN schedule for a 7,000-student campus (resource rooms + therapy + counselor rooms + assessment rooms) and clearly label it as “Design Assumptions (Non-TBC)” so it’s audit-safe.

If you want the next step, tell me which output format you prefer:

- A single “Space Program” table (rooms, qty, unit area, total area), or

- Separated by blocks (KG block, Grades 1–4 boys/girls, Grades 5–12 boys/girls), which is usually easier for masterplanning.

Sources

##### You said:

add as non TBC for SEN and also I need Net to Gross and I need a Table and Estimated Population per Section etc

###### ChatGPT said:

Thought for 2m 32s

According to TBC’s EnglishManual.pdf (Minimum Design Criteria – Category A: Riyadh), I’ve added (1) SEN as Non-TBC assumptions, (2) Net-to-Gross factors, and (3) a table showing the 7,000-student extrapolation, with population per section and classroom counts aligned to your co-ed/segregation rule.

## Net-to-Gross (N:G) factors to use (Planning Assumption – Non-TBC)

TBC gives net area rules (m²/student, worked examples) but does not give a net-to-gross multiplier in the extracted sections. So use these as planning factors:

- Academic blocks (classrooms + corridors + stairs + toilets + stores): 1.40 – 1.55 (recommend 1.45) (Non-TBC)

- Specialist / high-servicing areas (labs, SEN, clinic): 1.50 – 1.70 (recommend 1.60) (Non-TBC)

- Whole school campus grossing (all buildings + common spaces + MEP rooms): typically higher and handled at masterplan level (Non-TBC)

Formula: Gross Area = Net Area × N:G

## A) 7,000 Students – Estimated Population + Sections + Classrooms (with your segregation rule)

Your rule applied:

- Co-ed: Nursery, Reception, KG, Grade-1

- Separate boys/girls classrooms: Grade-2 to Grade-12

- Classroom cap in TBC for Elementary/Intermediate/Secondary: ≤ 25 students/classroom

EnglishManual

- KG area rule: 2.5 m²/child (Category A)

EnglishManual

Extrapolation basis: Your current 5,263 → target 7,000 (scale factor ≈ 1.33).
(I preserved each grade’s current boys/girls ratio to scale.)

### Classroom count result

- If we ignore gender split: 298 rooms

- With your Grade-2–12 separation: ✅ 304 classrooms

### Table: population & classrooms by grade (7,000 target)

| Grade | 7,000 Target Students (Boys/Girls/Total) | Section Size Basis | Boys Sections | Girls Sections | Total Rooms | Est. Students per Room |
| --- | --- | --- | --- | --- | --- | --- |
| Nursery (Co-ed) | 88 / 74 / 162 | ~20 | – | – | 9 | 18.0 |
| Reception (Co-ed) | 194 / 199 / 393 | ~20 | – | – | 20 | 19.7 |
| KG (Co-ed) | 270 / 240 / 510 | ~20 | – | – | 26 | 19.6 |
| Grade-1 (Co-ed) | 262 / 262 / 524 | 25 max EnglishManual | – | – | 21 | 25.0 |
| Grade-2 (Separate) | 304 / 251 / 555 | 25 max EnglishManual | 13 | 11 | 24 | 23.1 avg |
| Grade-3 (Separate) | 274 / 256 / 530 | 25 max EnglishManual | 11 | 11 | 22 | 24.1 avg |
| Grade-4 (Separate) | 277 / 295 / 572 | 25 max EnglishManual | 12 | 12 | 24 | 23.8 avg |
| Grade-5 (Separate) | 286 / 275 / 561 | 25 max EnglishManual | 12 | 11 | 23 | 24.4 avg |
| Grade-6 (Separate) | 302 / 321 / 623 | 25 max EnglishManual | 13 | 13 | 26 | 24.0 avg |
| Grade-7 (Separate) | 313 / 258 / 571 | 25 max EnglishManual | 13 | 11 | 24 | 23.8 avg |
| Grade-8 (Separate) | 242 / 264 / 506 | 25 max EnglishManual | 10 | 11 | 21 | 24.1 avg |
| Grade-9 (Separate) | 243 / 223 / 466 | 25 max EnglishManual | 10 | 9 | 19 | 24.6 avg |
| Grade-10 (Separate) | 235 / 203 / 438 | 25 max EnglishManual | 10 | 9 | 19 | 23.1 avg |
| Grade-11 (Separate) | 154 / 183 / 337 | 25 max EnglishManual | 7 | 8 | 15 | 22.5 avg |
| Grade-12 (Separate) | 134 / 117 / 251 | 25 max EnglishManual | 6 | 5 | 11 | 22.8 avg |

✅ Total classrooms (with separation): 304

## B) Room Types + Net Sizes (TBC) + Grossed Sizes (using N:G)

Below is a space-program table you can copy into Excel.
Where TBC provides a worked example, I used the example’s net area.

### Core teaching rooms (TBC net)

- Elementary classroom example net area (25 students): 43.12 m²

EnglishManual

- General science lab: 1 per 10 classrooms

EnglishManual

and has its own size criteria (e.g., Elementary 1.9 m²/student).

EnglishManual

- Learning Resources Center (examples):

  - Elementary: 75.1 m²

EnglishManual

  - Intermediate: 74.70 m²

EnglishManual

  - Secondary: 88.60 m²

EnglishManual

- Multi-purpose room (examples):

  - Intermediate: 45.90 m²

EnglishManual

  - Secondary: 51 m²

EnglishManual

### Space table (net + gross)

Assumed N:G: 1.45 academic, 1.60 specialist (Non-TBC)

| Space Type | Qty (7,000 plan) | Net Area (m²) | N:G | Gross/Room (m²) | Basis |
| --- | --- | --- | --- | --- | --- |
| Standard Classroom (Gr 1–12) | 304 minus KG/Rec/Nur | 43.12 | 1.45 | 62.5 | TBC classroom example EnglishManual |
| KG / Reception classroom | (Rec+KG rooms) 46 | = 2.5 m²/child × planned class size | 1.45 | varies | TBC 2.5 m²/child EnglishManual |
| Nursery Activity room | 9 | = 1.8 m²/child × planned class size | 1.45 | varies | TBC Nursery 1.8 EnglishManual |
| General Science Lab (Elementary) | see note below | (e.g., 1.9×25 + board strip) | 1.60 | varies | Ratio: 1/10 classrooms EnglishManual + size basis EnglishManual |
| Learning Resources Center – Elementary | Boys wing + Girls wing + (optional co-ed) | 75.1 | 1.45 | 108.9 | TBC example EnglishManual |
| Learning Resources Center – Intermediate | Boys + Girls | 74.70 | 1.45 | 108.3 | TBC example EnglishManual |
| Learning Resources Center – Secondary | Boys + Girls | 88.60 | 1.45 | 128.5 | TBC example EnglishManual |
| Multi-purpose – Intermediate | Boys + Girls | 45.90 | 1.45 | 66.6 | TBC example EnglishManual |
| Multi-purpose – Secondary | Boys + Girls | 51.0 | 1.45 | 74.0 | TBC example EnglishManual |
| Toilets (student clusters) | per block | 1.35 each (std) / 3.15 (accessible) | 1.45 | 2.0 / 4.6 | TBC toilet areas EnglishManual |
| Outdoor court/pitch | site-wide | 300 m² elem / 400 m² secondary | n/a | n/a | TBC minima EnglishManual EnglishManual |

### Science lab quantities (recommended with your segregation)

Because Grades 2–12 are gender-separated, it’s safer (approval-wise) to plan separate labs per boys wing and girls wing. TBC gives the ratio 1 science lab per 10 classrooms

EnglishManual

.

Using your classroom counts by wing (from the 7,000 plan):

- Elementary Boys (Gr2–6): 61 classrooms → 7 science labs

- Elementary Girls (Gr2–6): 58 classrooms → 6 science labs

- Intermediate Boys (Gr7–9): 33 classrooms → 4 science labs

- Intermediate Girls (Gr7–9): 27 classrooms → 3 science labs

- Secondary Boys (Gr10–12): 23 classrooms → 3 science labs

- Secondary Girls (Gr10–12): 22 classrooms → 3 science labs

✅ Total science labs (planning): 26

(If you want to minimize count, we can centralize labs and do timetable separation — but that depends on how strict operational segregation needs to be.)

## C) SEN / Inclusion Rooms (Non-TBC Assumption)

TBC pages we extracted don’t specify SEN room counts/sizes, so below is a clear “Non-TBC / best practice” schedule.

### Recommended SEN suite distribution (for 7,000 students)

Assumption: ~8–10% students need some learning support, smaller subset needs therapy rooms.

| SEN Space (Non-TBC) | Qty | Suggested Net (m²) | N:G | Gross/Room (m²) | Notes |
| --- | --- | --- | --- | --- | --- |
| SEN Coordinator Office | 2 | 12 | 1.60 | 19.2 | one per boys/girls campus wing |
| Resource Room (small group) | 10 | 25 | 1.60 | 40.0 | distributed: Early Years 2, Elem boys 3, Elem girls 3, Secondary boys 1, Secondary girls 1 |
| 1:1 / Assessment Room | 8 | 12 | 1.60 | 19.2 | quiet testing/counsel sessions |
| Speech & Language Therapy | 4 | 16 | 1.60 | 25.6 | shared within each gender wing where needed |
| Occupational Therapy | 2 | 20 | 1.60 | 32.0 | one per boys/girls |
| Sensory Room | 2 | 24 | 1.60 | 38.4 | one per boys/girls |
| Counsellor Room | 4 | 12 | 1.60 | 19.2 | junior + senior split per wing |
| SEN Storage (assistive tools) | 4 | 10 | 1.60 | 16.0 | near resource rooms |

If you want, I can also add SEN-adjacent rooms typically requested in approvals (clinic rooms, nurse stations, isolation room, etc.) as a separate non-TBC block.

## What I can do next (fast)

If you want this in one clean Excel-ready table, I can output it as:

- Table 1: Grade-by-grade 7,000 plan (boys/girls, sections, rooms)

- Table 2: Space program (room type, qty, net, gross, basis: TBC vs Non-TBC)

Just tell me your preferred default N:G (e.g., 1.45 academic / 1.60 specialist), or I’ll lock those as the baseline in the table.

Sources

##### You said:

how many staff rooms and academic blocs adn other or are they covered in net

###### ChatGPT said:

Good question — staff rooms, academic blocks, admin, corridors, stairs, toilets, MEP rooms are NOT “covered in net” unless you explicitly include them in your net schedule. In most school briefs:

- Net Area (NIA) = usable rooms (classrooms, labs, LRC, offices, staff rooms, SEN rooms, etc.)

- Gross Area (GIA) = Net + circulation + walls/structure + toilets (if not counted in net) + plant/MEP + shafts + stair cores + risers + stores + cleaners + etc.

TBC mostly gives room-level minimums and some examples; it does not “auto-include” staff rooms or academic blocks into those classroom net numbers. So you must plan them explicitly in your schedule OR treat them as part of the Grossing factor.

## 1) Are staff rooms covered in Net?

No (unless you add them as net rooms).
In your space program, you should list staff rooms as separate net items, then apply N:G to get gross.

## 2) How many “academic blocks” do we need?

An academic block is a planning construct (not a TBC-defined room). It is basically:

- A set of classrooms + toilets + stairs + staff support + stores + breakout/collaboration corners + circulation.

So blocks are not “covered in net”; they’re how you group the rooms.

For 7,000 students you have 304 classrooms (with your segregation rule). Practically, you don’t want a single mega-block; you want multiple blocks by stage + gender.

### Recommended academic block structure (planning best practice)

Using a sensible block size of 24–30 classrooms per block:

- Co-ed Early Years (Nursery+Reception+KG+G1)

  - Rooms: 9 + 20 + 26 + 21 = 76 classrooms

  - Blocks: 3 blocks (≈ 24–26 rooms each)

- Elementary (G2–G4) Boys + Girls

  - G2–G4 total rooms: G2 24 + G3 22 + G4 24 = 70 rooms

  - Split by gender ≈ 35 + 35

  - Blocks: 2 blocks boys + 2 blocks girls (≈ 17–18 rooms each)
(These can be 2-storey blocks.)

- Upper (G5–G12) Boys + Girls

  - G5–G12 rooms: 23+26+24+21+19+19+15+11 = 158 rooms

  - Split by gender ≈ 79 + 79

  - Blocks: 3 blocks boys + 3 blocks girls (≈ 26 rooms each)

✅ Total Academic Blocks (recommended): ~13 blocks
(3 co-ed + 4 elementary gender blocks + 6 upper gender blocks)

You can tighten to 10–11 blocks if you make bigger blocks, but it increases circulation, supervision complexity, and peak crowding.

## 3) How many Staff Rooms are needed?

This depends on whether you mean:

- Teacher workrooms (shared staff rooms)

- Staff lounges / break rooms

- Department staff rooms (e.g., Science, Languages)

### A) Teacher workrooms / staff rooms (most typical)

A good school planning rule is:

- 1 staff room per academic block, plus

- 1 dedicated staff room per Early Years cluster (because EYFS staffing patterns differ)

So with the ~13 academic blocks above:

- 13 staff rooms (baseline)

If you want fewer but larger:

- 1 staff room per 40–60 teachers (planning assumption)

For a 7,000-student school, staff often lands around 1:12 to 1:18 (varies by curriculum and specialist load). That could mean ~390–580 staff total (teachers + assistants). Teachers might be ~300–450.

Using “1 per 50 teachers” gives:

- ~6–9 large staff rooms (but you still typically need distributed workrooms near blocks)

✅ Recommendation: 12–14 staff rooms distributed (smaller) + 2 large central lounges.

### B) Department staff rooms (optional but helpful)

- Science Dept staff room: 1 boys + 1 girls = 2

- Math/English Dept rooms: 1 each per campus wing (boys/girls) = 2–4
These can be treated as non-essential early and added later.

## 4) Are “other rooms” covered by Net-to-Gross?

They are covered only if you choose to cover them via the grossing factor.

Two approaches:

### Approach 1 (Recommended): Explicit Net Schedule

You list as net:

- staff rooms, admin offices, clinics, SEN rooms, meeting rooms, security, store rooms, etc.
Then apply N:G for gross.

### Approach 2: Partial Net + Heavy Gross Factor

You list only teaching rooms as net, and you “hide” most support spaces in gross factor.
This is riskier because approvals and briefing become vague.

✅ For SMC / TBC submissions, Approach 1 is cleaner.

## 5) Quick “Support Spaces” checklist you should add to Net schedule

These are usually not in the classroom net and must be planned:

Per Academic Block

- Staff room / workroom

- HOD office (optional)

- Student toilets (boys/girls)

- Cleaner closet

- General storage

- Print/copy nook (optional)

- Sick bay / first aid point (optional)

Central / Whole School

- Administration suite (principal, deputy, admissions, finance)

- Clinic (male/female nursing if required by segregation)

- Central staff lounge

- Meeting rooms

- Exam office / records

- Security control

- Parent waiting area

- IT room / server room

- Reprographics

- Maintenance workshop + stores

- MEP plant rooms (electrical LV room, UPS, AHU rooms, pump room, etc.)

## What I’ll do next if you want it in a single table

I can produce an Excel-ready table with:

- Block count (13)

- Staff rooms (12–14 baseline)

- Recommended staff room net size (e.g., 40–60 m² net each depending on teacher count)

- Admin suite net allowances

- Clinic/SEN included (clearly tagged “TBC vs Non-TBC”)

If you tell me whether you want smaller distributed staff rooms or fewer large staff rooms, I’ll lock the counts and sizes.

##### You said:

principal and it aand other support fuctions rooms etc and if you can createa this a paart of the full table as net adn then we do gross in one final full table

###### ChatGPT said:

Thought for a couple of seconds

Got it. Below is a Support Functions Net Area Schedule you can plug into the full Net table first (clearly marking Non-TBC), and then we can apply Net→Gross in the final consolidated table.

Because the TBC English Manual sections we used earlier focus mainly on teaching spaces (classrooms/labs/LRC/multipurpose/toilets ratios), these support rooms are generally not prescribed there as exact m², so I’m listing them as Non-TBC (planning assumptions / best practice).

## Support & Administration Space Schedule (NET) – Non-TBC Assumptions

Planning basis: 7,000 students, gender separation from Grade-2 onward, multiple academic blocks.

### A) Leadership and Academic Administration

| Function | Space | Qty | Typical Net Area Each (m²) | Net Total (m²) | Notes |
| --- | --- | --- | --- | --- | --- |
| School leadership | Principal Office | 1 | 20 | 20 | Near reception/admin |
| School leadership | Vice Principal / Deputy (Boys campus) | 1 | 16 | 16 | If boys/girls operationally separated |
| School leadership | Vice Principal / Deputy (Girls campus) | 1 | 16 | 16 |  |
| Leadership support | Executive Assistant / Secretary | 2 | 10 | 20 | 1 per principal/deputy cluster |
| Governance | Board/SMC Meeting Room | 1 | 30 | 30 | 12–16 people |
| Academic | Academic Coordinator / Head of Curriculum | 2 | 14 | 28 | 1 boys + 1 girls (optional, but recommended) |
| Academic | Exam Cell / Assessment Office | 2 | 18 | 36 | 1 boys + 1 girls |
| Academic | Records / Student Files Room (secure) | 2 | 20 | 40 | Adjacent to admin |
| Academic | HOD Offices (cluster) | 8 | 10 | 80 | 4 boys + 4 girls (Science/Math/English/ICT etc.) |

Subtotal A (NET): ~322 m²

### B) Admissions, Finance, HR, Parent Services

| Function | Space | Qty | Net Area Each (m²) | Net Total (m²) | Notes |
| --- | --- | --- | --- | --- | --- |
| Front-of-house | Reception / Welcome Desk | 2 | 25 | 50 | 1 per main entry / campus |
| Parent services | Parent Waiting Lounge | 2 | 40 | 80 | Comfortable, controlled access |
| Parent services | Interview / Meeting Rooms | 6 | 12 | 72 | Admissions, counsellor, discipline, etc. |
| Admissions | Admissions Office | 2 | 18 | 36 | 1 boys + 1 girls |
| Finance | Cashier / Fee Collection (secure) | 2 | 16 | 32 | Controlled queue space nearby |
| Finance | Finance Office (Back office) | 2 | 24 | 48 | Accounts team workspace |
| HR | HR Office | 2 | 18 | 36 | 1 boys + 1 girls or centralized |
| General admin | Admin Open Office / Clerical | 2 | 40 | 80 | Shared admin staff work area |
| Compliance | Document Control / Archive | 2 | 16 | 32 | Can be combined with records if needed |

Subtotal B (NET): ~498 m²

### C) Student Wellbeing, Clinic, Safeguarding, SEN Interface

(You already asked SEN separately; these are support-wellbeing functions.)

| Function | Space | Qty | Net Area Each (m²) | Net Total (m²) | Notes |
| --- | --- | --- | --- | --- | --- |
| Health | Clinic / Nurse Room (Boys) | 1 | 20 | 20 | With exam bed |
| Health | Clinic / Nurse Room (Girls) | 1 | 20 | 20 |  |
| Health | Isolation Room (Boys) | 1 | 10 | 10 | For fever/temporary isolation |
| Health | Isolation Room (Girls) | 1 | 10 | 10 |  |
| Safeguarding | Student Counselling Room (Boys) | 2 | 12 | 24 |  |
| Safeguarding | Student Counselling Room (Girls) | 2 | 12 | 24 |  |
| Wellbeing | School Psychologist / Social Worker | 2 | 12 | 24 | If included |
| Inclusion interface | SEN / Inclusion Admin Office | 2 | 12 | 24 | Links to SEN suite |

Subtotal C (NET): ~156 m²

### D) Staff Support (Workrooms, Lounges, Collaboration)

(These are not “inside classroom net”; include them explicitly.)

| Function | Space | Qty | Net Area Each (m²) | Net Total (m²) | Notes |
| --- | --- | --- | --- | --- | --- |
| Staff | Staff Workroom (distributed) | 12 | 45 | 540 | ~1 per academic block |
| Staff | Staff Lounge (large) | 2 | 90 | 180 | 1 boys + 1 girls |
| Staff | Staff Pantry / Tea Point | 6 | 10 | 60 | 1 per 2 blocks |
| Staff | Staff Prayer / Quiet Room | 2 | 30 | 60 |  |
| Staff | Staff Changing / Lockers (M) | 1 | 40 | 40 | If required |
| Staff | Staff Changing / Lockers (F) | 1 | 40 | 40 | If required |
| Collaboration | Teacher Training / PD Room | 2 | 45 | 90 | 1 boys + 1 girls or shared |

Subtotal D (NET): ~1,010 m²

### E) IT / AV / Data / Security Control (Your “IT and other support functions”)

| Function | Space | Qty | Net Area Each (m²) | Net Total (m²) | Notes |
| --- | --- | --- | --- | --- | --- |
| IT | IT Office / Helpdesk | 2 | 18 | 36 | 1 per campus wing |
| IT | Server Room / MDF (Main) | 2 | 20 | 40 | Boys + girls or two buildings |
| IT | IDF / Comms Rooms (distributed) | 12 | 6 | 72 | ~1 per block/floor cluster |
| AV | AV / Media Store & Repair | 2 | 16 | 32 | Equipment storage/charging |
| Security | Security Control Room (CCTV) | 2 | 14 | 28 |  |
| Security | Gatehouse / Guard Room | 4 | 8 | 32 | Main gates, boys/girls gates |

Subtotal E (NET): ~240 m²

### F) Operations, Maintenance, Stores, Cleaning, Logistics

| Function | Space | Qty | Net Area Each (m²) | Net Total (m²) | Notes |
| --- | --- | --- | --- | --- | --- |
| Ops | Operations Manager Office | 2 | 12 | 24 |  |
| FM | Maintenance Workshop | 2 | 35 | 70 | 1 boys + 1 girls |
| FM | General Stores (central) | 2 | 60 | 120 | Furniture/books/exam etc. |
| FM | Cleaning Store/Janitor Closet | 20 | 3 | 60 | 1 per cluster/floor |
| Logistics | Receiving / Delivery Holding | 2 | 25 | 50 | Near service yard |
| Waste | Waste Holding (internal) | 2 | 20 | 40 | Separate from student circulation |

Subtotal F (NET): ~364 m²

## Net Support Total

Add the subtotals:

- A: 322

- B: 498

- C: 156

- D: 1,010

- E: 240

- F: 364

✅ Total Support Functions (NET) ≈ 2,590 m² (Non-TBC planning allowance)

This is in addition to:

- Teaching spaces (classrooms/labs/LRC/multipurpose) (TBC-guided)

- SEN suite (Non-TBC, which we already drafted)

- Toilets (some TBC ratios exist; but we can keep toilets as “included in block grossing” if you prefer)

## How we convert this into the final “Full Table”

We’ll do it in two tables (exactly as you requested):

### Table 1 — FULL NET SCHEDULE

- Teaching rooms (TBC)

- Specialist rooms (TBC where defined)

- SEN (Non-TBC)

- Support functions (Non-TBC) ✅ (the schedule above)

### Table 2 — FULL GROSS SCHEDULE

Apply N:G factors (you can change them any time):

- Academic / admin areas: 1.45

- Specialist / high-servicing (IT, clinic, SEN, labs): 1.60

- Large assembly / multipurpose / canteen (if any): 1.50–1.65 (depending on structure/MEP)

Gross = Net × N:G

## Quick question-free next move (I’ll assume this unless you say otherwise)

I will proceed using:

- 1.45 for Admin/Staff rooms

- 1.60 for IT/Clinic/SEN/Server/Workshops

If you want different factors, just tell me the multipliers and I’ll swap them.

If you want, paste (or confirm) whether you are planning:

- One combined admin building with controlled access to boys/girls wings, or

- Two parallel admin nodes (boys + girls), or

- Hybrid (one central + small satellite).

Either way, the table above already works; it just changes distribution and a few quantities.

##### You said:

whata about security rooma and all other rooms like cafeteria and canteen adn then we ahve sports facilites and

###### ChatGPT said:

Absolutely — those are not covered in “net” unless you list them, and they’re big drivers of gross area + site area. Below I’m adding them to the Support Functions NET schedule (mostly Non-TBC assumptions unless TBC specifies a minimum).

I’ll keep it table-first, so you can paste straight into your master Net schedule.

## G) Security, Visitor Control, Student Discipline (NET)

(Mostly Non-TBC; TBC doesn’t generally prescribe room m² for these in the sections we extracted.)

| Function | Space | Qty | Typical Net Area Each (m²) | Net Total (m²) | Notes |
| --- | --- | --- | --- | --- | --- |
| Security | Main Security Control Room (CCTV/monitoring) | 1 | 20 | 20 | Central campus |
| Security | Boys Wing Security Room | 1 | 14 | 14 | Near boys entry |
| Security | Girls Wing Security Room | 1 | 14 | 14 | Near girls entry |
| Security | Guardhouse / Gatehouse (main gates) | 4 | 8 | 32 | Separate gates & service entry |
| Security | Visitor Screening / Reception Checkpoint | 2 | 18 | 36 | With controlled access |
| Discipline / welfare | Student Discipline / Reflection Room | 2 | 12 | 24 | One per wing |
| Discipline / welfare | Lost & Found / Student Services Counter | 2 | 10 | 20 | Optional |

Subtotal G (NET): ~160 m²

## H) Cafeteria / Dining / Canteen / Food Services (NET)

(Non-TBC planning assumptions. Capacity is an operations choice.)

### Key planning decision (implicit): do you serve in shifts?

For 7,000 students, cafeterias are almost always multi-shift, otherwise the dining hall becomes enormous.

Baseline assumption (Non-TBC): cafeteria seats ~25% of total population at one time (multi-shift).

- Seating at a time ≈ 7,000 × 25% = 1,750 seats

Typical net area allowances (Non-TBC):

- Dining seating: 1.2–1.4 m² per seat (use 1.3)

- Servery: 0.25–0.35 m² per seat

- Kitchen / prep / wash: 0.35–0.55 m² per seat (depends if cooked vs reheat)

- Storage/cold rooms: add separately

### Net schedule (recommended)

| Function | Space | Qty | Typical Net Area (m²) | Net Total (m²) | Notes |
| --- | --- | --- | --- | --- | --- |
| Student dining | Dining Hall (Boys) | 1 | 1,100 | 1,100 | ~850 seats in shifts |
| Student dining | Dining Hall (Girls) | 1 | 1,100 | 1,100 | ~850 seats in shifts |
| Food service | Servery (Boys) | 1 | 180 | 180 | Queue + serving |
| Food service | Servery (Girls) | 1 | 180 | 180 |  |
| Food prep | Kitchen / Reheat Kitchen | 2 | 220 | 440 | One per wing or central split |
| Hygiene | Dishwash / Wash-up | 2 | 80 | 160 |  |
| Storage | Dry store | 2 | 50 | 100 |  |
| Storage | Cold room / freezer | 2 | 25 | 50 |  |
| Staff | Catering staff room + lockers | 2 | 20 | 40 |  |
| Waste | Food waste holding | 2 | 15 | 30 | Separate from main waste |

Subtotal H (NET): ~3,380 m²
(We can shrink this if you choose smaller seating %, or convert to “distributed canteens” per block.)

## I) Sports Facilities (NET + Site Area)

TBC references minimum outdoor pitch/court sizes in the educational facilities section:

- Outdoor pitch minimum 300 m² (elementary)

- Secondary outdoor court minimum 400 m²

But for a 7,000-student campus you’ll need more than minima. Here’s a practical program:

### Outdoor sports (site area, not building net)

| Facility | Qty | Recommended Size | Notes |
| --- | --- | --- | --- |
| Football field (full size) | 1 | ~7,000–8,000 m² | If you want proper field; otherwise smaller |
| Multi-sport courts (basketball/volleyball) | 8–12 | ~420–500 m² each | Separate boys/girls blocks by scheduling or physical separation |
| Shaded PE courts | 4–6 | ~420 m² each | Riyadh climate benefit |
| Running track (optional) | 1 | 200m–400m loop | Depends on land |

### Indoor / covered PE (NET areas)

| Function | Space | Qty | Typical Net Area Each (m²) | Net Total (m²) | Notes |
| --- | --- | --- | --- | --- | --- |
| PE indoor | Covered Multi-purpose Hall (Boys) | 1 | 900 | 900 | Assembly + PE |
| PE indoor | Covered Multi-purpose Hall (Girls) | 1 | 900 | 900 |  |
| Changing | Boys changing + showers | 1 | 160 | 160 | Include WC/shower |
| Changing | Girls changing + showers | 1 | 160 | 160 |  |
| Storage | Sports equipment store | 4 | 25 | 100 | Near courts/halls |
| Staff | PE staff office | 2 | 12 | 24 |  |
| First aid | Sports first-aid point | 2 | 10 | 20 | Near halls |

Subtotal I (NET): ~2,264 m²
(You can reduce hall size if you rely on outdoor-only + staggered timetables.)

## Updated Support NET Total (adding your new items)

Previous Support NET (A–F) ≈ 2,590 m²
Add:

- G Security ≈ 160 m²

- H Cafeteria ≈ 3,380 m²

- I Sports indoor ≈ 2,264 m²

✅ New Support + Common Facilities NET ≈ 8,394 m² (Non-TBC planning allowances)

This is before SEN suite (Non-TBC) and before your teaching/specialist rooms (TBC-based).

## How this rolls into the “Final Full Table”

We’ll do exactly what you asked:

- Full NET schedule = Teaching (TBC) + Specialist (TBC) + SEN (Non-TBC) + Support/Common (Non-TBC).

- Final Gross schedule = apply N:G:

  - Admin/academic/common: 1.45

  - Specialist high-MEP (kitchen, server, clinic, IT, SEN, halls): 1.60–1.70

  - Sports halls: 1.60 (typical)

## One operational assumption I will lock (unless you tell me otherwise)

For cafeteria, I assumed 25% seating at a time (multi-shift).
If you want 20% or 30%, I can recalc the dining areas instantly.

If you reply with: “Use 20% seating” or “Use 30% seating”, I’ll update the cafeteria row totals and keep everything consistent.

##### You said:

what about exam and adutoriium etc for 300 and open area sitting for 2000 that is covered attrium etc..

###### ChatGPT said:

Yes — exam halls, auditoriums, and an atrium / open seating “commons” are not covered unless you add them as NET rooms (or you intentionally “hide” them inside a big grossing factor, which I don’t recommend for approvals/briefing).

Below is the NET schedule add-on for:

- Exam facilities

- Auditorium (300 seats)

- Atrium / Learning Commons / Open seating for 2,000

These are Non-TBC planning assumptions (the TBC English Manual sections we’ve been using don’t prescribe explicit m² for these spaces in the extracted parts).

## J) Exams & Assessment Facilities (NET) — Non-TBC

Planning approach: You can do exams via classrooms, but for a 7,000-student school you’ll typically want a central exam hall + supporting rooms.

| Function | Space | Qty | Typical Net Area Each (m²) | Net Total (m²) | Notes |
| --- | --- | --- | --- | --- | --- |
| Exams | Main Exam Hall | 1 | 750 | 750 | ~300 candidates at exam spacing (see note below) |
| Exams | Exam Stores (secure) | 2 | 25 | 50 | Papers, stationery, sealed storage |
| Exams | Invigilator Briefing / Control | 1 | 20 | 20 | Near exam hall |
| Exams | Printing / Reprographics (secure) | 1 | 30 | 30 | Controlled access |
| Exams | Candidate Waiting / Holding | 2 | 60 | 120 | Separate boys/girls holding |
| Exams | Accessible exam room (small) | 2 | 20 | 40 | SEN/access needs |
| Exams | Toilets (near exam hall) | 2 | 30 | 60 | Can be counted under toilet clusters instead |

Subtotal J (NET): ~1,070 m²

Exam hall sizing note (Non-TBC):

- 300 candidates typically needs 2.0–2.5 m² per candidate depending on desk spacing, aisle widths, and invigilator circulation.

- 300 × 2.5 = 750 m² (the value I used).
If you want it 500 candidates, tell me and I’ll resize.

## K) Auditorium (300 seats) (NET) — Non-TBC

This is a classic assembly/auditorium package: seating bowl + stage + backstage + control + lobby.

| Function | Space | Qty | Typical Net Area Each (m²) | Net Total (m²) | Notes |
| --- | --- | --- | --- | --- | --- |
| Performance | Auditorium seating bowl (300 seats) | 1 | 420 | 420 | ~1.4 m²/seat incl aisles |
| Performance | Stage | 1 | 120 | 120 | School productions |
| Performance | Backstage / Green room | 2 | 20 | 40 | Boys/girls or shared |
| Storage | Stage / props store | 1 | 30 | 30 |  |
| AV | Control room / projection | 1 | 15 | 15 |  |
| FOH | Lobby / foyer | 1 | 160 | 160 | Pre-function + queueing |
| FOH | Ticket / reception counter | 1 | 10 | 10 | Optional |
| Support | Auditorium toilets (nearby) | 2 | 30 | 60 | Or include in toilet clusters |

Subtotal K (NET): ~855 m²

Sizing note (Non-TBC):

- If you want a “proper” auditorium with wider seats, more aisle width, and acoustic wall thickness, the grossing factor will rise (we’ll capture that in the Gross table).

## L) Atrium / Learning Commons / Open Seating for 2,000 (NET) — Non-TBC

This is the big one. The key question is whether the “2,000 seating” is:

- Loose gathering / standing + mixed seating, or

- Fixed auditorium-style seats (much bigger).

You said: “open area sitting for 2000… covered atrium etc.”
So I’m treating it as informal commons (soft seating / steps / bleachers / study tables), not fixed theater seating.

### Recommended sizing logic (Non-TBC)

- Informal commons: 0.8–1.2 m² per person depending on density and furniture.

- For comfortable seating + circulation: use 1.0 m²/person.

So:

- 2,000 × 1.0 = 2,000 m² NET (commons only)

Add supporting elements: breakout rooms, small meeting pods, AV, storage, toilets adjacency.

| Function | Space | Qty | Typical Net Area Each (m²) | Net Total (m²) | Notes |
| --- | --- | --- | --- | --- | --- |
| Commons | Atrium / Covered Learning Commons | 1 | 2,000 | 2,000 | Open seating for ~2,000 |
| Learning | Breakout rooms (glass) | 8 | 25 | 200 | Small group / project rooms |
| Learning | Seminar rooms | 4 | 45 | 180 | Flexible teaching / clubs |
| Support | Commons storage | 2 | 15 | 30 | Furniture / event kit |
| AV | Commons AV / control closet | 2 | 8 | 16 | Screens, audio rack |
| Support | Info desk / supervision point | 2 | 10 | 20 | Line-of-sight control |

Subtotal L (NET): ~2,446 m²

Important Riyadh note (Non-TBC):
A covered atrium of this size has heavy HVAC / smoke control / egress implications. It’s doable, but grossing will be higher (we’ll treat it as specialist N:G 1.60–1.70 in the final table).

## New add-on total (NET)

- Exams (J): ~1,070 m²

- Auditorium 300 (K): ~855 m²

- Atrium/Commons 2,000 (L): ~2,446 m²

✅ Added NET = ~4,371 m²

## How we’ll gross these in the “final full table”

When we do the Gross table:

- Exams + Auditorium: use N:G = 1.60 (specialist/high-servicing)

- Atrium/Commons: use N:G = 1.65 (bigger volume + smoke control + structure + services)

(You can override these multipliers any time.)

### If you want me to lock it perfectly (no back-and-forth)

I’ll assume:

- Exam hall sized for 300 candidates at 750 m² NET

- Commons density 1.0 m²/person → 2,000 m² NET

- Auditorium 300 seats with stage, lobby, backstage as above

Next step: I can merge A–L into one single Master NET table (with columns: TBC vs Non-TBC, qty, unit net, total net, N:G, unit gross, total gross).

##### You said:

Any other things missing in the school if not can we createa. line by line table in a cavas .. first we will have section by section , leanring and sports and academic support and amdin etc.. or whatever categories and then we create in the end a final table specifiyng hwo the cirulation works and the numbers

###### ChatGPT said:

Done — I created a Canvas draft called:

“PISES New Building – Space Program (NET First) + Circulation Logic (Draft)”

It’s organized exactly how you asked:

- Section-by-section (teaching by stage)

- Learning & labs

- Sports

- Academic support + admin

- IT / security / operations

- Cafeteria

- Exams + auditorium + atrium/commons

- Then a circulation logic section and a missing checklist.

Next, tell me which of these you want included (just reply with the checkbox list, e.g., “include 1,2,5,6”):

- Prayer halls / musalla (students & staff; boys/girls)

- Ablution areas

- School store/bookshop

- Uniform shop

- Transport office + driver lounge/toilets

- Parent/community training rooms

- Science prep rooms + chemical store (secondary)

- Fire command center / civil defense room

- Anything else you already know you want (e.g., swimming, gym, clinic expansion, counseling suite enlargement)

Once you confirm, I’ll lock all TBD unit areas, then produce the final two master tables:

- Full NET table (line-by-line)

- Full GROSS table with your Net→Gross factors + a circulation/numbers summary (zones, blocks, peak loads).

##### You said:

school store and bookshop and transport office and uniform shop. science prep room and chemcical sotre and fire comand center. swwiming pool and clinic if not already

###### ChatGPT said:

Done — I’ve added all the items you listed into the Canvas as line-by-line NET rows, and tagged them correctly:

✅ Added to the NET table (Non-TBC planning allowances):

- School Store + Bookshop

- Uniform shop

- Transport office + driver lounge + driver toilets

- Science prep rooms

- Chemical store (ventilated / controlled access)

- Fire Command Center / Civil Defense room

- Swimming pool (pool hall + plant/filtration + changing + storage + lifeguard/first-aid)

- Expanded clinic (treatment room, waiting, medical store, accessible WC) — in addition to nurse + isolation

In the Canvas, you’ll find them under:

- Section 5 (Admin) → bookshop + uniform shop

- Section 7 (Ops/Security/IT) → transport suite + fire command center

- Section 3 (Labs/Specialist) → prep rooms + chemical store

- Section 4 (Wellbeing) → clinic expansion

- Section 9 (Sports) → swimming pool suite

Next step (as you requested): we’ll produce the final master table that shows:

- NET totals by category

- Grossing factors by category

- Final GROSS totals

- A short circulation/numbers summary (zones, blocks, peak flows)

To do that cleanly, I’ll now convert the remaining TBD unit areas in the Canvas (mainly: lab unit areas, computer/language labs, some art atelier items) into fixed numbers — using TBC wherever it’s explicitly defined and using Non-TBC where TBC is silent.

##### You said:

why some is TBD

###### ChatGPT said:

Good question — and this is important strategically.

They are marked TBD (To Be Determined) for only one reason:

Because those specific unit areas must be taken directly from the TBC English Manual tables, and I didn’t want to “invent” a number where TBC gives a defined minimum.

So:

- If TBC gives a clear unit (e.g., 2.5 m²/student for KG) → I fixed it.

- If TBC gives a worked example (e.g., 43.12 m² classroom) → I fixed it.

- If TBC gives a ratio but not a fixed unit size in the extracted section (e.g., science labs per 10 classrooms but lab size varies by stage) → I left it TBD so we don’t misstate a regulatory requirement.

This is a compliance discipline decision, not uncertainty.

## Where the TBDs currently exist (and why)

### 1️⃣ Science Lab Unit Area

TBC specifies:

- Ratios (e.g., 1 lab per 10 classrooms)

- Different m²/student for Elementary / Intermediate / Secondary labs

But:

- Exact lab net area depends on:

  - Student capacity (20? 24? 25?)

  - Fixed benches vs island benches

  - Prep adjacency included or separate

I can now fix them stage-by-stage.

### 2️⃣ Computer / Language Labs

Same issue:

- TBC defines minimum lab sizes by student count,

- But we must decide:

  - 20-student lab?

  - 25-student lab?

  - Shared or per gender?

Once we lock capacity, I’ll fix the number.

### 3️⃣ Some Early Years detailed sub-rooms

Nursery:

- Activity room = 1.8 m²/child

- Bedroom = 0.9 m²/child

We need to lock:

- Final class size (20? 22?)

Once you confirm class size cap for EY, I’ll fill all those automatically.

# Let’s Lock It Properly Now

To eliminate all TBDs, I need 5 decisions from you:

- Classroom capacity for G2–G12

  - Keep at 25?
(You were using 25 already — I’ll assume 25 unless you say otherwise.)

- Science lab capacity

  - 24 students?

  - 25 students?

- Computer / language lab capacity

  - 20 students?

  - 25 students?

- Early Years class cap

  - 20?

  - 22?

- Do you want labs duplicated per gender always, or allow shared labs with timetable separation?

If you reply with something like:

25 classroom, 24 lab, 20 computer, 20 EY, labs separated

I will:

- Remove all TBDs

- Produce a fully numeric NET table

- Then generate the complete GROSS table

- Then calculate:

  - Total built-up area

  - Site requirement estimate

  - Circulation core requirements

We’re now at the stage where the numbers become real.

##### You said:

based upon this strenth how many teachers are needed and if we ahve a school which is exxtended that will also confimr the rooms so we have max like 25?

###### ChatGPT said:

Thought for 1m 9s

With your current strength table (5,263 students), you already have 304 teaching sections / classrooms (Nursery→G12). If you scale to a 7,000-student campus and cap classes at max 25 students, you should plan on ~280 teaching sections (7,000 ÷ 25 = 280) as the theoretical minimum.

But because you have Early Years (smaller / more support), gender separation from G2 onward, and timetable constraints, the safe planning number is:

### 1) Classroom cap (Yes, 25 is the max)

TBC explicitly states: “maximum number of students per classroom shall not exceed 25 students” for Elementary, Intermediate, and Secondary.

EnglishManual

EnglishManual

EnglishManual

So 25 is the correct max to use for mainstream classrooms (G1–G12).

# A) Teachers needed (best-practice planning based on your structure)

## A1) “Direct classroom teachers” (homeroom/primary)

These are the non-negotiable ones because every section needs a class teacher.

- Nursery + Reception + KG: 55 sections → 55 lead teachers

- Grade 1–4: (G1 21 + G2 24 + G3 22 + G4 24 = 91 sections) → 91 class teachers

Subtotal (class/lead teachers) = 146 teachers

Note: In Early Years you will also need teaching assistants (not “teachers” but essential staffing). Typically 1 assistant per EY section → ~55 assistants (plus floaters).

## A2) Subject teachers for Grade 5–12

From your table, Grade 5–12 student population ≈ 3,753 students.

A good planning ratio for subject-based secondary in a school like yours (with labs, electives, segregation, activity rooms) is:

- 1 teacher per 16 students for G5–G12 (teaching staff only, not assistants)

So: 3,753 / 16 ≈ 235 subject teachers

Subtotal (G5–G12 subject teachers) ≈ 235

## A3) Specialist teachers shared across school

Even if some are embedded in secondary, you still plan a dedicated pool for:

- PE / swimming (lifesaving + PE specialists)

- Art, music, drama

- ICT / robotics

- Library / learning resources center

- SEN / learning support (you already want SEN rooms)

- Counselors / wellbeing (especially with segregated wings)

A realistic planning allowance for your scale is:

- ~60 to 90 specialist teachers (depending on how rich you want the timetable)

## ✅ Total teacher headcount (for your current 5,263 strength)

Putting it together:

- Class/lead teachers (EY + G1–G4): 146

- Subject teachers (G5–G12): ~235

- Specialists: ~60–90

### Total teachers ≈ 441 to 471

That range is what you should use for budgeting and space planning (staff rooms, workrooms, HOD offices).

# B) If you expand to 7,000 students (and keep max 25)

Scale factor from 5,263 → 7,000 = 1.33×

So teacher count scales roughly to:

### ~590 to 630 teachers (for 7,000 students)

Why a range? Because:

- Early years staffing scales more heavily (assistants),

- segregation increases duplication (boys/girls wings),

- enrichment (labs, electives) increases specialist load.

# C) Does expanding confirm room count?

Yes — because teacher numbers and room count are linked by timetable:

- You can’t reduce classrooms below the number of sections

- And you can’t reduce specialist rooms (labs, ICT, etc.) below timetable demand

Also note: TBC has a hard ratio for science labs in secondary:

- “Single science lab for every 10 classrooms” (secondary).

EnglishManual

So when your classroom count rises, the minimum lab count rises automatically.

## Quick rule-of-thumb you can use immediately

For planning at 7,000 students with max 25:

- Classrooms (minimum theoretical): 280

- Classrooms (practical with EY + segregation + buffering): ~300–320

And that aligns closely with your current 304 sections already.

If you want, I can now produce one clean staffing table:

By stage (EY, Primary, Secondary)

- Students

- Sections

- Class teachers

- Subject teachers

- Specialist teachers

- Assistants (non-teaching)

- Total adults

…and that table will directly drive:

- staff rooms count

- teacher workroom sizes

- HOD offices

- locker / printing / planning rooms

Sources

##### You said:

yes and update the estiamtes for area need and room count and sizes

###### ChatGPT said:

Thought for 4m 4s

### 1) Class size cap

Yes — for the mainstream grades, you should plan max 25 students per classroom (that’s the correct cap to use for your extended school and it directly drives the room count).

## 2) Teachers needed (updated for 7,000 students, aligned to your section structure)

Because you’re running a sectioned school (each section needs a teacher assigned and a timetable built around it), the most practical way is to estimate teachers in three layers:

### A) “Lead/Class Teachers” (fixed by number of sections)

These are the teachers you can’t avoid because they anchor the class.

- Early Years (Nursery + Reception + KG): 9 + 20 + 26 = 55 lead teachers

- Primary (Grade 1–4): 21 + 24 + 22 + 24 = 91 class teachers

✅ Lead/Class Teachers total = 146

Separate note (not teachers): Early Years will also require Teaching Assistants — typically 1 per EY section → ~55 assistants (+ floaters).

### B) Subject Teachers (Grade 5–12)

Your scaled Grade 5–12 population at 7,000 is about 3,755 students (scaled from your current totals).

A realistic planning ratio for subject-based grades in your model (separate boys/girls wings + labs + electives) is:

- 1 subject teacher per 16 students (teaching staff only)

So: 3,755 ÷ 16 ≈ 235 subject teachers

✅ G5–G12 subject teachers ≈ 235

### C) Specialist Teachers (shared load)

Across the whole school (or duplicated per gender where required):

- PE + swimming

- Art / music / drama

- ICT / robotics / makerspaces

- Library/LRC staffing

- SEN/inclusion teachers (specialists)

- Counselors (if treated as teaching staff in your model)

For 7,000, a safe planning range is:

✅ Specialist teachers: ~70–100

### ✅ Total Teachers (7,000 plan)

- Lead/Class teachers: 146

- Subject teachers (G5–G12): ~235

- Specialists: ~70–100

✅ Total Teachers ≈ 451 to 481

If you want a single budget number: ~470 teachers is a good planning baseline.

# 3) Updated room count + sizes (7,000 plan, max 25)

## A) Rooms (confirmed from your scaled structure + segregation rule)

- Total teaching sections / rooms (Nursery→G12): ~304

- Standard classrooms (Grade 1–12 only) = 249 rooms

  - Unit Net (standard classroom): 43.12 m² (TBC example-based)

- Reception + KG rooms = 46 rooms

  - Unit Net = 2.5 m²/child × class size

  - If class size = 20 → 50 m² each

- Nursery (activity + sleeping model)

  - Activity = 1.8 m²/child × class size

  - Bedroom = 0.9 m²/child × class size

### Teaching NET area (now updated with fixed assumptions)

Using these locked assumptions (for now):

- Reception/KG class size = 20

- Nursery class size = 20

- Grade 1–12 max = 25 (and classroom net 43.12 already aligns to 25)

Confirmed teaching net (core):

- Standard classrooms: 249 × 43.12 = 10,736.9 m²

- Reception + KG: 46 × 50 = 2,300 m²

- Nursery activity: 9 × 36 = 324 m²

- Nursery bedrooms: 9 × 18 = 162 m²

- KG in-class toilets (if you keep them): 46 × 1.8 = 82.8 m²

✅ Core Teaching NET ≈ 13,606 m² (before labs, art, ICT, etc.)

## B) Support + shared facilities (you asked to include these in NET)

From what you’ve already approved in the Canvas (admin + IT + security + cafeteria + sports halls + exams + auditorium + atrium/commons + new adds like pool, bookshop, transport, fire command etc.):

✅ Support/Common NET (current working total) ≈ 14,893 m²
(This includes your new adds: bookshop, uniform shop, transport suite, fire command center, science prep & chemical stores, expanded clinic items, and the swimming pool suite.)

# 4) Updated total area (NET → GROSS)

### NET total (current working)

- Core Teaching NET (confirmed): ~14,276 m² (teaching + LRC + known multipurpose; excludes labs still being finalized)

- Support/Common NET: ~14,893 m²

✅ Working NET subtotal ≈ 29,169 m²
(Still excludes a few specialist teaching items that are in your Canvas as TBD: certain lab unit areas and computer/language labs until we lock capacity per lab.)

## Gross area (planning)

Use these grossing factors (as we discussed):

- Academic/admin: 1.45

- Specialist/high-services (kitchen, pool plant, atrium, halls, labs): 1.65

A safe consolidated result for your masterplan budgeting is:

✅ Gross Built-Up Area (BUA) likely ≈ 45,000 to 55,000 m²

- The lower end assumes efficient stacking + shared specialist rooms.

- The upper end assumes stronger duplication (boys/girls) + generous commons and specialist labs.

# 5) What I need from you to eliminate the last TBDs and finalize the “one final master table”

Just reply with these 3 numbers:

- Reception/KG class size: keep 20 or make 22?

- Science lab capacity: 24 or 25 students per lab?

- Computer lab capacity: 20 or 25?

Once you answer those, I will:

- remove all remaining TBDs,

- update the Canvas to a fully numeric NET table,

- then produce the final combined table with:

  - NET totals by category,

  - Grossing by category,

  - final GROSS totals,

  - circulation logic + block counts (how movement works + peaks + cores).

##### You said:

all capacity is 25

###### ChatGPT said:

Perfect — all capacities locked at 25 ✅

### What this immediately fixes (no more guesswork)

It locks the unit NET areas for Early Years where TBC uses m²/child rules:

- Reception + KG classroom NET = 2.5 m²/child × 25 = 62.5 m² / room

- Nursery activity NET = 1.8 m²/child × 25 = 45.0 m² / room

- Nursery bedroom NET = 0.9 m²/child × 25 = 22.5 m² / room

### Updated Early Years NET totals (with your section counts)

- Nursery activity: 9 × 45.0 = 405.0 m²

- Nursery bedroom: 9 × 22.5 = 202.5 m²

- Reception: 20 × 62.5 = 1,250.0 m²

- KG: 26 × 62.5 = 1,625.0 m²

✅ Early Years core NET (those four lines) = 3,482.5 m²
(+ any EY common/play, stores, toilets, etc. you keep in the schedule.)

## Updated classroom requirement (7,000 @ max 25)

- The theoretical minimum is 280 classrooms (7,000 ÷ 25).

- Your structure + segregation + Early Years model keeps you around the ~300–320 range.

- You already had a working number of ~304 sections/classrooms in the plan, which remains a solid target.

✅ Planning classroom count stays: ~304 classrooms

## Updated teacher requirement (7,000 @ capacity 25)

Locking capacity doesn’t reduce teachers much because your system is section-driven (especially with boys/girls separation).

A good planning baseline remains:

✅ ~470 teachers (planning budget figure)
Range: ~450–500 depending on specialist richness (labs, electives, PE/swim, SEN staffing).

## About the Canvas update

I attempted to update the Canvas automatically to replace the “TBD” values, but the update failed because one of the search patterns didn’t match the exact text in the document (the regex couldn’t find the sentence).

No content was changed in the Canvas in that attempt.

If you want, I can still update it cleanly by applying only the line-item replacements (EY-01, EY-02, EY-04, EY-05) without touching narrative lines — and then you’ll see those numbers filled in.

## Next (to finalize the full NET + GROSS table)

Now that capacity = 25 everywhere, the only remaining “TBD” items are not capacity-related — they’re TBC-defined specialist room unit areas that must be copied exactly from the English Manual tables (science labs, ICT/language labs, etc.), plus a few Non-TBC items we can lock.

If you say “go ahead”, I’ll:

- Fill every remaining TBD with a numeric unit area (TBC where defined; Non-TBC clearly tagged where not),

- Produce the final master table:

  - NET (line-by-line)

  - Grossed table (by category multiplier)

  - Circulation summary (blocks, cores, peak loads, how the separation works)

##### You said:

go ahead

###### ChatGPT said:

Thought for 5m 55s

Pises New Building – Space Program (net First) + Circulation Logic (draft)

# PISES New Building – Space Program (NET First) + Circulation Logic (Draft)

(Category A – Riyadh; Teaching-space minimums per TBC where stated; Support/SEN/Commons/Auditorium/Cafeteria etc. as Non-TBC planning assumptions.)

## 0) How to use this table

- Phase 1 (NET): We list rooms line-by-line (qty × unit net area).

- Phase 2 (GROSS): We apply Net→Gross factors by category (academic/admin vs specialist high-services).

- Phase 3 (Circulation & logistics): We validate movement: entries, separation, vertical cores, peak loads.

Columns explained

- Basis: TBC (from English Manual) or Non-TBC (assumption/best practice).

- Campus Split: Co-ed (Nursery–G1) vs Boys and Girls (G2–G12) as per your rule.

## 1) Student population + classroom count (Target 7,000)

- Total classrooms (with G2–G12 gender separation): 304 rooms

- Co-ed classrooms (Nursery–G1): 76 rooms

- Boys classrooms (G2–G12): ~114 rooms

- Girls classrooms (G2–G12): ~114 rooms

(Classroom sizing and caps are handled in the Teaching section below.)

## 2) TEACHING SPACES (by Stage) — NET schedule

### 2.1 Early Years (Nursery + Reception + KG) — Co-ed

| Line | Space | Qty | Unit Net (m²) | Total Net (m²) | Basis | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| EY-01 | Nursery Activity Room | 9 | 45.0 | 405.0 | TBC | 1.8 m²/child × 25 |
| EY-02 | Nursery Bedroom | 9 | 22.5 | 202.5 | TBC | 0.9 m²/child × 25 |
| EY-03 | Nursery Feeding Room | 3 | 4.5 | 13.5 | TBC | 1.5 m² per 10 infants → (25 ÷10)≈3 ×1.5 = 4.5 m² |
| EY-04 | Reception Classroom | 20 | 62.5 | 1,250.0 | TBC | 2.5 m²/child × 25 |
| EY-05 | KG Classroom | 26 | 62.5 | 1,625.0 | TBC | 2.5 m²/child × 25 |
| EY-06 | EY Outdoor court (per class) | 46 | (½ class area) | — | TBC | Outdoor court = ½ classroom area (site area). |
| EY-07 | EY Toilets inside class | 46 | 1.8 | 82.8 | TBC | One toilet inside each KG class (as per TBC kindergarten). |
| EY-08 | EY Learning/Play commons | 3 | 120 | 360 | Non-TBC | One per EY block (shared messy play/reading). |
| EY-09 | EY Store (general) | 3 | 12 | 36 | Non-TBC | Per EY block |

### 2.2 Grade 1–4 (G1 co-ed; G2–G4 separated) — NET

| Line | Space | Qty | Unit Net (m²) | Total Net (m²) | Basis | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| P-01 | Grade-1 Classroom (Co-ed) | 21 | 43.12 | 905.5 | TBC | Classroom example net for 25 students. |
| P-02 | Grade-2 Classrooms (Boys) | 13 | 43.12 | 560.6 | TBC | Split by gender |
| P-03 | Grade-2 Classrooms (Girls) | 11 | 43.12 | 474.3 | TBC | Split by gender |
| P-04 | Grade-3 Classrooms (Boys) | 11 | 43.12 | 474.3 | TBC |  |
| P-05 | Grade-3 Classrooms (Girls) | 11 | 43.12 | 474.3 | TBC |  |
| P-06 | Grade-4 Classrooms (Boys) | 12 | 43.12 | 517.4 | TBC |  |
| P-07 | Grade-4 Classrooms (Girls) | 12 | 43.12 | 517.4 | TBC |  |
| P-08 | Primary Multi-purpose room | 4 | 42.5 | 170.0 | TBC | One per primary block (planning) |
| P-09 | Primary LRC / Library | 2 | 75.1 | 150.2 | TBC | One per boys/girls primary wing (recommend). |
| P-10 | Primary Art Atelier | 4 | 41.9 | 167.6 | TBC | Arts atelier (Elementary, Category A) example-based net |

### 2.3 Grade 5–12 (Separated Boys/Girls) — NET

| Line | Space | Qty | Unit Net (m²) | Total Net (m²) | Basis | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| S-01 | Classrooms (Boys G5–G12) | 79 | 43.12 | 3,406.5 | TBC | 25-student baseline classroom net |
| S-02 | Classrooms (Girls G5–G12) | 79 | 43.12 | 3,406.5 | TBC |  |
| S-03 | Intermediate LRC (Boys) | 1 | 74.70 | 74.7 | TBC |  |
| S-04 | Intermediate LRC (Girls) | 1 | 74.70 | 74.7 | TBC |  |
| S-05 | Secondary LRC (Boys) | 1 | 88.60 | 88.6 | TBC |  |
| S-06 | Secondary LRC (Girls) | 1 | 88.60 | 88.6 | TBC |  |
| S-07 | Multi-purpose (Intermediate Boys) | 1 | 45.90 | 45.9 | TBC |  |
| S-08 | Multi-purpose (Intermediate Girls) | 1 | 45.90 | 45.9 | TBC |  |
| S-09 | Multi-purpose (Secondary Boys) | 1 | 51.0 | 51.0 | TBC |  |
| S-10 | Multi-purpose (Secondary Girls) | 1 | 51.0 | 51.0 | TBC |  |

## 3) LEARNING LABS, SPECIALIST ROOMS & MAKER SPACES — NET schedule

(Science + ICT labs use TBC minimum unit net areas for Category A, capacity 25. Quantities are validated against classroom counts + timetable; where TBC does not mandate a ratio, the qty rule is marked Non-TBC.)

| Line | Space | Qty | Unit Net (m²) | Total Net (m²) | Basis | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| L-01 | General Science Lab – Primary (Boys) | 7 | 60.1 | 420.7 | TBC | Category A (Elementary) example-based net; capacity 25 |
| L-02 | General Science Lab – Primary (Girls) | 6 | 60.1 | 360.6 | TBC |  |
| L-03 | Science Lab – Intermediate (Boys) | 4 | 62.96 | 251.84 | TBC | Category A (Intermediate) example-based net |
| L-04 | Science Lab – Intermediate (Girls) | 3 | 62.96 | 188.88 | TBC |  |
| L-05 | Physics/Chem/Bio Labs – Secondary (Boys) | 6 | 69.9 | 419.4 | TBC | Category A (Secondary) example-based net (subject labs) |
| L-06 | Physics/Chem/Bio Labs – Secondary (Girls) | 6 | 69.9 | 419.4 | TBC |  |
| L-07 | Science Prep Room (per lab cluster) | 6 | 18 | 108 | Non-TBC | Shared between 2–3 labs |
| L-08 | Chemical Store (secondary; ventilated) | 2 | 12 | 24 | Non-TBC | Boys + Girls; controlled access |
| ICT-01 | Computer/Language Lab – Primary (Boys) | 3 | 60.1 | 180.3 | TBC (unit) / Non-TBC (qty) | Unit per TBC; qty planned per primary wing |
| ICT-02 | Computer/Language Lab – Primary (Girls) | 3 | 60.1 | 180.3 | TBC (unit) / Non-TBC (qty) |  |
| ICT-03 | Computer/Language Lab – Secondary (Boys) | 5 | 69.9 | 349.5 | TBC (unit) / Non-TBC (qty) | Unit per TBC; qty planned per wing |
| ICT-04 | Computer/Language Lab – Secondary (Girls) | 5 | 69.9 | 349.5 | TBC (unit) / Non-TBC (qty) |  |
| MAK-01 | Maker / Robotics Lab | 2 | 120 | 240 | Non-TBC | One per boys/girls secondary (or shared via scheduling) |
| ART-S1 | Art Studios (Secondary) | 2 | 90 | 180 | Non-TBC | Larger than ateliers |
| MUS-01 | Music / Drama Room | 2 | 80 | 160 | Non-TBC | Coordinate with auditorium back-of-house |

## 4) SEN & WELLBEING SUITE — NET schedule

All SEN items are Non-TBC (explicitly tagged).

| Line | Space | Qty | Unit Net (m²) | Total Net (m²) | Basis | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| SEN-01 | SEN Coordinator Office | 2 | 12 | 24 | Non-TBC | Boys + Girls |
| SEN-02 | Resource Room (small group) | 10 | 25 | 250 | Non-TBC | Distributed by stage |
| SEN-03 | 1:1 Assessment Room | 8 | 12 | 96 | Non-TBC | Quiet testing |
| SEN-04 | Speech & Language Therapy | 4 | 16 | 64 | Non-TBC |  |
| SEN-05 | Occupational Therapy | 2 | 20 | 40 | Non-TBC |  |
| SEN-06 | Sensory Room | 2 | 24 | 48 | Non-TBC |  |
| SEN-07 | Counsellor Rooms | 4 | 12 | 48 | Non-TBC |  |
| SEN-08 | SEN Storage | 4 | 10 | 40 | Non-TBC |  |

Wellbeing (non-SEN):

| Line | Space | Qty | Unit Net (m²) | Total Net (m²) | Basis | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| WB-01 | Clinic / Nurse Room (Boys) | 1 | 20 | 20 | Non-TBC | Exam bed + handwash |
| WB-02 | Clinic / Nurse Room (Girls) | 1 | 20 | 20 | Non-TBC | Exam bed + handwash |
| WB-05 | Treatment / Minor Procedures (Boys) | 1 | 12 | 12 | Non-TBC |  |
| WB-06 | Treatment / Minor Procedures (Girls) | 1 | 12 | 12 | Non-TBC |  |
| WB-07 | Clinic Waiting (controlled) | 2 | 12 | 24 | Non-TBC |  |
| WB-08 | Medical Store | 2 | 6 | 12 | Non-TBC | Lockable |
| WB-09 | Accessible Clinic WC | 2 | 3.15 | 6.3 | Non-TBC | Coordinate with toilet clusters |

| WB-03 | Isolation Room (Boys) | 1 | 10 | 10 | Non-TBC | | | WB-04 | Isolation Room (Girls) | 1 | 10 | 10 | Non-TBC | |

## 5) ACADEMIC SUPPORT & ADMINISTRATION — NET schedule

| Line | Space | Qty | Unit Net (m²) | Total Net (m²) | Basis | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| ADM-01 | Reception / Welcome Desk | 2 | 25 | 50 | Non-TBC | Main + secondary entry |
| ADM-02 | Principal Office | 1 | 20 | 20 | Non-TBC |  |
| ADM-03 | Deputy/Vice Principal (Boys) | 1 | 16 | 16 | Non-TBC |  |
| ADM-04 | Deputy/Vice Principal (Girls) | 1 | 16 | 16 | Non-TBC |  |
| ADM-05 | Executive Assistants / Secretaries | 2 | 10 | 20 | Non-TBC |  |
| ADM-06 | Admissions Offices | 2 | 18 | 36 | Non-TBC |  |
| ADM-07 | Finance (Back office) | 2 | 24 | 48 | Non-TBC |  |
| ADM-08 | Cashier / Fee Collection (secure) | 2 | 16 | 32 | Non-TBC |  |
| ADM-09 | HR Office | 2 | 18 | 36 | Non-TBC |  |
| ADM-10 | Meeting / Interview Rooms | 6 | 12 | 72 | Non-TBC |  |
| ADM-11 | Board/SMC Meeting Room | 1 | 30 | 30 | Non-TBC |  |
| ADM-12 | Exam Cell / Assessment Office | 2 | 18 | 36 | Non-TBC |  |
| ADM-13 | Records / Archive (secure) | 2 | 20 | 40 | Non-TBC |  |
| ADM-14 | HOD Offices (cluster) | 8 | 10 | 80 | Non-TBC |  |
| ADM-15 | School Store / Bookshop | 1 | 80 | 80 | Non-TBC | Retail + text books + stationery (controlled access) |
| ADM-16 | Uniform Shop | 1 | 60 | 60 | Non-TBC | Can be adjacent to bookshop |

## 6) STAFF SUPPORT — NET schedule

| Line | Space | Qty | Unit Net (m²) | Total Net (m²) | Basis | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| ST-01 | Staff Workroom (distributed) | 12 | 45 | 540 | Non-TBC | ~1 per academic block |
| ST-02 | Staff Lounge (large) | 2 | 90 | 180 | Non-TBC | Boys + Girls |
| ST-03 | Staff Pantry / Tea point | 6 | 10 | 60 | Non-TBC |  |
| ST-04 | Teacher Training / PD Room | 2 | 45 | 90 | Non-TBC |  |
| ST-05 | Staff lockers / changing (M) | 1 | 40 | 40 | Non-TBC | Optional |
| ST-06 | Staff lockers / changing (F) | 1 | 40 | 40 | Non-TBC | Optional |

## 7) IT / AV / SECURITY / OPERATIONS — NET schedule

| Line | Space | Qty | Unit Net (m²) | Total Net (m²) | Basis | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| IT-01 | IT Office / Helpdesk | 2 | 18 | 36 | Non-TBC |  |
| IT-02 | Server Room / MDF | 2 | 20 | 40 | Non-TBC |  |
| IT-03 | IDF / Comms Rooms | 12 | 6 | 72 | Non-TBC | ~per block/cluster |
| SEC-01 | Main Security Control (CCTV) | 1 | 20 | 20 | Non-TBC |  |
| SEC-02 | Wing Security Room (Boys) | 1 | 14 | 14 | Non-TBC |  |
| SEC-03 | Wing Security Room (Girls) | 1 | 14 | 14 | Non-TBC |  |
| SEC-04 | Gatehouses / Guard rooms | 4 | 8 | 32 | Non-TBC |  |
| OPS-01 | Maintenance Workshop (Boys) | 1 | 35 | 35 | Non-TBC |  |
| OPS-02 | Maintenance Workshop (Girls) | 1 | 35 | 35 | Non-TBC |  |
| OPS-03 | Central Stores (Boys) | 1 | 60 | 60 | Non-TBC |  |
| OPS-04 | Central Stores (Girls) | 1 | 60 | 60 | Non-TBC |  |
| OPS-05 | Janitor Closets | 20 | 3 | 60 | Non-TBC |  |
| OPS-06 | Receiving / delivery holding | 2 | 25 | 50 | Non-TBC |  |
| OPS-07 | Waste holding (internal) | 2 | 20 | 40 | Non-TBC |  |
| SAF-01 | Fire Command Center / Civil Defense Room | 1 | 18 | 18 | Non-TBC | Near main entry; coordinate with fire alarm panel & access control |
| TRN-01 | Transport Office (Routes, buses, permits) | 1 | 18 | 18 | Non-TBC | Near arrival/departure zone |
| TRN-02 | Driver Lounge | 1 | 25 | 25 | Non-TBC | Seating + rest |
| TRN-03 | Driver Toilets | 2 | 6 | 12 | Non-TBC | Male/female if needed |

## 8) FOOD SERVICES — NET schedule (Cafeteria / Canteen)

(Non-TBC; the major driver is whether you run multi-shift dining.)

| Line | Space | Qty | Unit Net (m²) | Total Net (m²) | Basis | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| F-01 | Dining Hall (Boys) | 1 | 1,100 | 1,100 | Non-TBC | Multi-shift assumption |
| F-02 | Dining Hall (Girls) | 1 | 1,100 | 1,100 | Non-TBC |  |
| F-03 | Servery (Boys) | 1 | 180 | 180 | Non-TBC |  |
| F-04 | Servery (Girls) | 1 | 180 | 180 | Non-TBC |  |
| F-05 | Kitchen / Prep / Wash-up | 2 | 300 | 600 | Non-TBC | Can be centralised |
| F-06 | Dry store | 2 | 50 | 100 | Non-TBC |  |
| F-07 | Cold rooms | 2 | 25 | 50 | Non-TBC |  |
| F-08 | Catering staff room/lockers | 2 | 20 | 40 | Non-TBC |  |
| F-09 | Food waste holding | 2 | 15 | 30 | Non-TBC |  |

## 9) SPORTS & PE — NET + Site schedule

### 9.1 Indoor / covered PE (NET)

| Line | Space | Qty | Unit Net (m²) | Total Net (m²) | Basis | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| SP-01 | Covered Multi-purpose Sports Hall (Boys) | 1 | 900 | 900 | Non-TBC | PE + events |
| SP-02 | Covered Multi-purpose Sports Hall (Girls) | 1 | 900 | 900 | Non-TBC |  |
| SP-03 | Changing + showers (Boys) | 1 | 160 | 160 | Non-TBC |  |
| SP-04 | Changing + showers (Girls) | 1 | 160 | 160 | Non-TBC |  |
| SP-05 | Sports storage | 4 | 25 | 100 | Non-TBC |  |
| SP-06 | PE staff office | 2 | 12 | 24 | Non-TBC |  |
| SP-07 | Sports first-aid point | 2 | 10 | 20 | Non-TBC |  |

### 9.2 Swimming pool (NET + Site)

| Line | Space | Qty | Unit Net (m²) | Total Net (m²) | Basis | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| SW-01 | Swimming Pool Hall (25m x 12.5m water) | 1 | 1,200 | 1,200 | Non-TBC | Includes pool surround, deck, lifeguard point (net) |
| SW-02 | Pool Plant / Filtration Room | 1 | 120 | 120 | Non-TBC | High-services (grossing higher) |
| SW-03 | Pool Changing + showers (Boys) | 1 | 180 | 180 | Non-TBC |  |
| SW-04 | Pool Changing + showers (Girls) | 1 | 180 | 180 | Non-TBC |  |
| SW-05 | Pool Store | 1 | 25 | 25 | Non-TBC |  |
| SW-06 | Pool First-aid / Lifeguard room | 1 | 12 | 12 | Non-TBC |  |

### 9.3 Outdoor sports (SITE AREA)

| Line | Facility | Qty | Suggested Area | Basis | Notes |
| --- | --- | --- | --- | --- | --- |
| OUT-01 | Outdoor pitch minimum | — | 300 m² | TBC | Elementary minimum |
| OUT-02 | Outdoor court minimum | — | 400 m² | TBC | Secondary minimum |
| OUT-03 | Football field (full) | 1 | ~7,000–8,000 m² | Non-TBC | If desired |
| OUT-04 | Multi-sport courts | 8–12 | ~420–500 m² each | Non-TBC | Split/schedule boys vs girls |
| OUT-05 | Shaded PE courts | 4–6 | ~420 m² each | Non-TBC | Riyadh climate |

## 10) EXAMS, AUDITORIUM, ATRIUM/COMMONS — NET schedule

| Line | Space | Qty | Unit Net (m²) | Total Net (m²) | Basis | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| EX-01 | Main Exam Hall (~300 candidates) | 1 | 750 | 750 | Non-TBC | Adjustable |
| EX-02 | Exam store (secure) | 2 | 25 | 50 | Non-TBC |  |
| EX-03 | Reprographics / printing (secure) | 1 | 30 | 30 | Non-TBC |  |
| EX-04 | Candidate holding rooms | 2 | 60 | 120 | Non-TBC | Boys + Girls |
| AUD-01 | Auditorium (300 seats) seating bowl | 1 | 420 | 420 | Non-TBC |  |
| AUD-02 | Stage | 1 | 120 | 120 | Non-TBC |  |
| AUD-03 | Backstage / green rooms | 2 | 20 | 40 | Non-TBC |  |
| AUD-04 | Lobby / foyer | 1 | 160 | 160 | Non-TBC |  |
| ATR-01 | Atrium / Learning Commons (2,000) | 1 | 2,000 | 2,000 | Non-TBC | Informal seating |
| ATR-02 | Breakout rooms (glass) | 8 | 25 | 200 | Non-TBC |  |
| ATR-03 | Seminar rooms | 4 | 45 | 180 | Non-TBC | Clubs/project |

## 11) What might still be missing (Checklist)

Included per your instruction (now added to NET schedule):

- ✅ School store / bookshop

- ✅ Uniform shop

- ✅ Transport office + driver lounge/toilets

- ✅ Science prep rooms + chemical store (secondary)

- ✅ Fire command center

- ✅ Swimming pool

- ✅ Expanded clinic (if not already)

## 12) Circulation Logic (for the final combined table)

### 12.1 Campus zoning

- Zone A (Co-ed): Nursery–G1

- Zone B (Girls): G2–G12 + girls specialist rooms

- Zone C (Boys): G2–G12 + boys specialist rooms

- Shared / controlled: Auditorium, atrium/commons, central admin (if chosen), central kitchen (if chosen)

### 12.2 Separation rules

- Separate student entries for Boys/Girls (G2–G12)

- Shared spaces use timetable separation or controlled access routes

### 12.3 Peak movement checks (to be calculated)

We will compute:

- Students per zone

- Stair/core counts per block

- Corridor widths for peaks

- Lunch shifts & queuing area

- Arrival/departure bus flow

## 13) Next step

- Convert all former TBD items into fixed unit net areas (DONE for EY + Labs + Ateliers).

- Produce the Final Master Table (NET + GROSS) and a short circulation summary.

## 14) Working area totals (NET → GROSS) — Target 7,000 students

(These totals use the room schedule above; any future changes in qty (labs, ateliers, multipurpose rooms, dining strategy) will update the totals.)

### 14.1 NET totals (working)

- Teaching (EY + Primary + Secondary): ~15,720 m² NET

- Labs + ICT + Specialist teaching rooms: ~3,832 m² NET

- SEN + Wellbeing: ~736 m² NET

- Admin: ~672 m² NET

- Staff support: ~950 m² NET

- IT/Security/Ops: ~641 m² NET

- Food services: ~3,380 m² NET

- Sports (incl. pool suite): ~3,981 m² NET

- Exams + Auditorium + Atrium/Commons: ~4,070 m² NET

✅ Total NET (working) ≈ 33,983 m²

### 14.2 Grossing (planning)

Use grossing multipliers by category:

- Academic/Admin/Staff/SEN: × 1.45

- High-services (Labs, Food, Sports, Auditorium/Atrium): × 1.65

- Ops/Back-of-house (workshops/stores): × 1.55

✅ Total GROSS / BUA (working) ≈ 52,400 m²

*(This BUA excludes external site items: outdoor pitches/courts, shaded play areas, drop-off loops, parking, and perimeter security.)

Done ✅

I’ve now locked capacity = 25 across all teaching + lab spaces, and I have:

- Filled all major TBDs in the Canvas using the TBC (English Manual) unit net rules for Category A (Riyadh) where the manual provides them:

  - Nursery activity 45.0 m²

  - Nursery bedroom 22.5 m²

  - Reception + KG classroom 62.5 m²

  - Science Lab (Primary/Elementary) 60.1 m²

  - Science Lab (Intermediate) 62.96 m²

  - Science Labs (Secondary subject labs) 69.9 m²

  - Computer/Language labs (same as science lab per stage)

  - Arts atelier (Primary/Elementary) 41.9 m²

- Removed duplicated repeated blocks in the Canvas (you had repeats in Sections 3 / 5 / 7).

- Added NET → GROSS totals (working) at the end.

### Your updated working totals (now visible in Canvas)

✅ Total NET (working) ≈ 33,983 m²
✅ Total GROSS / BUA (working) ≈ 52,400 m²

This BUA still excludes external site-only items: outdoor pitches/courts, shaded yards, drop-off loops, parking, perimeter/security road, etc. (those stay as “site area”, not BUA).

If you want, next I can append a one-page “room count summary” (counts only) by Zone:

- Zone A (Co-ed EY–G1)

- Zone B (Girls G2–G12)

- Zone C (Boys G2–G12)
…and a quick “what drives the BUA up/down” sensitivity list (labs qty, dining strategy, atrium size, sports hall size).